Question 1A

$4=a

$5=b

- a = ( a < b );

main:	slt $4, $4, $5		#Sets $4[a]=1   if $4[a] is strictly  < than $5[b]

#If the condition a < b false, $4[a] = 0

- a = ( a > b);

main:	slt $4, $5, $4		#Sets $4[a]=1   if $5[b] is strictly < than $4[a]

#Otherwise, $4[a] = 0

- a = (a ==b);

main:	beq $4, $5, Equal	#Branches to Equal if $4[a] = $5[b]

#otherwise (if a != b) do this

ori $4, $0, 0		#$4[a] = 0

beq $4, $0, Exit	#Skips “Equal” to avoid overwriting a

#Branched here from 3001 if a=b; need to set a = 1

Equal:	ori $4, $0, 1

Exit:	#Rest of the program

- a = (a != b);

Main:	bne $4, $5, NotEqual	#Branches to NotEqual if $4[a] != $5[b]

#Otherwise, meaning a = b, need to reset a = 0

ori $4. $0, 0		#Reset a=0

beq $4, $0, Exit	#Skips next line to avoid overwriting a

#Branched here if a != b. Need to set a = 1

NotEqual:	ori $4, $0, 1		#Sets a = 1

Exit:	#Rest of the program

- a = (a <=b);

main: 	bne $4, $5, NotEqual	#Goes to NotEqual if a != b

#If a =b

ori $4, $0, 1		#Sets a = 1 if  a = b

bne $4, $0, Exit	#a = 1, Branch to Exit if a != 0

#Only way to get here is if a != b

NotEqual:	slt $4, $4, $5		#Sets a = 1 if a<b

Exit:	#Rest of the program

- a = (a >=b);

Main:	bne $4, $5, NotEqual	#Goes to NotEqual if a != b

#If a=b

ori $4, $0, 1		#Sets a = 1 if a = b

bne $4, $0, 6004	#a = 1; unconditional branch because a != 0

#If a !=b

NotEqual:	slt $4, $5, $4		#Sets a = 0 if b < a

Exit:	#Rest of the program

Question 2 A

This is what goes on in the stack. Further down, for Q2b, I have outlined the affect of the MIPS instructions. The pushes, since they can overwrite each other are coloured. The pops are done in order, from the top of the stack (lower address) to the bottom of it (higher address)

| -4 | Store $11=42 |  | 42 | POP $11 |
| --- | --- | --- | --- | --- |
| -8 | Store $10=arr[0] |  | arr[0] | POP $10 |
| -12 | Store $31=rtn main |  | rtn main | POP $31 |
| -16 | Store $13=17 | POP $18 | Store $18=22 | POP $13 |
| -20 | Store $14=12 | POP $17 | Store $17=12 | POP $14 |
| -24 |  |  |  |  |

So basically, we

Push $11 to stack,

Push $10 to stack,

Push $31 to stack,

Push $13 to stack,

Push $14 to stack.

Then we

Pop $17 from stack

Pop $18 from stack

Push $18 to stack

Push $17 to stack

Pop $14 from stack

Pop $13 from stack

Pop $31 from stack

Pop $10 from stack

Pop $11 from stack

Then we are back to ground level with an empty stack.

This is basically what happens chronologically in the registers:

|  | $31 | $sp | $10 | $11 | $12 | $13 | $14 | $15 | $16 | $17 | $18 | $2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| load $10=arr1 |  | 0 | arr[0] |  |  |  |  |  |  |  |  |  |
| $11=arr1[0] |  | 0 | arr[0] | 42 |  |  |  |  |  |  |  |  |
| $sp-8 |  | -8 | arr[0] | 42 |  |  |  |  |  |  |  |  |
| Store $11 @-4 |  | -8 | arr[0] | 42 |  |  |  |  |  |  |  |  |
| Store $10 @-8 |  | -8 | arr[0] | 42 |  |  |  |  |  |  |  |  |
| Jump to bar | rt main | -8 | arr[0] | 42 |  |  |  |  |  |  |  |  |
| $sp -4 | rt main | -12 | arr[0] | 42 |  |  |  |  |  |  |  |  |
| Store $31 | rt main | -12 | arr[0] | 42 |  |  |  |  |  |  |  |  |
| $12=adr arr1 | rt main | -12 | arr[0] | 42 | arr[0] |  |  |  |  |  |  |  |
| $12+8 | rt main | -12 | arr[0] | 42 | arr[2] |  |  |  |  |  |  |  |
| lw $13 = arr[2] | rt main | -12 | arr[0] | 42 | arr[2] | 22 |  |  |  |  |  |  |
| $12=arr[+1] | rt main | -12 | arr[0] | 42 | arr[3] | 22 |  |  |  |  |  |  |
| $14= arr[3] | rt main | -12 | arr[0] | 42 | arr[3] | 22 | 12 |  |  |  |  |  |
| $sp -4 | rt main | -16 | arr[0] | 42 | arr[3] | 22 | 12 |  |  |  |  |  |
| Store $13 @-16 | rt main | -16 | arr[0] | 42 | arr[3] | 22 | 12 |  |  |  |  |  |
| $sp -4 | rt main | -16 | arr[0] | 42 | arr[3] | 22 | 12 |  |  |  |  |  |
| Store $14 @ -20 | rt main | -20 | arr[0] | 42 | arr[3] | 22 | 12 |  |  |  |  |  |
| Jal to Foo |  |  |  |  |  |  |  |  |  |  |  |  |
| POP $17 Fr-20 | rt bar | -20 | arr[0] | 42 | arr[3] | 22 | 12 |  |  | 12 |  |  |
| POP $18 Fr-16 | rt bar | -20 | arr[0] | 42 | arr[3] | 22 | 12 |  |  | 12 | 22 |  |
| $sp +8 | rt bar | -12 | arr[0] | 42 | arr[3] | 22 | 12 |  |  | 12 | 22 |  |
| $15=arr[0] | rt bar | -12 | arr[0] | 42 | arr[3] | 22 | 12 | arr[0] |  | 12 | 22 |  |
| $15=arr[0+1] | rt bar | -12 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] |  | 12 | 22 |  |
| $16=arr[1] | rt bar | -12 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| $sp -8 | rt bar | -20 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| Store $17 @ -20 | rt bar | -20 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| Store $18 @ -16 | rt bar | -20 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| Jump back to $31 | rt bar | -20 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| POP $14 Fr-20 | rt bar | -20 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| Increm sp+4 | rt bar | -16 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| POP $13 Fr-16 | rt bar | -16 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| sp+4 | rt bar | -12 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| POP $31 Fr-12 | rt main | -12 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| sp+4 | rt main | -8 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| Jump to $31 | rt main | -8 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| POP $10 Fr-8 | rt main | -8 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| POP $11 Fr-4 | rt main | -8 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| sp + 8 | rt main | 0 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 |  |
| set $2=10 | rt main | 0 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 | 10 |

So at the end of the program,

| rt main | 0 | arr[0] | 42 | arr[3] | 22 | 12 | arr[1] | 32 | 12 | 22 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| $31 | $sp | $10 | $11 | $12 | $13 | $14 | $15 | $16 | $17 | $18 | $2 |

$11: Contains arr1[0] = 42

$13: Contains arr1[2] = 22

$14: Contains arr1[3] = 12

$16: Contains arr1[1] = 32

$17: Contains arr1[3] = 12

$18: Contains arr1[2] = 22

Q3 : Doing in 2 hours what a computer can do in 0.2 seconds : MIPE-Hex translation

Here is the assembly code translated in machine code, represented in hexedecimal

| Main | 200A 0020 |
| --- | --- |
|  | 2006 0000 |
| Main10 | 316C 0001 |
|  | 1180 0002 |
|  | 20C6 0001 |
| Main20 | 000b 5842 |
|  | 214A FFFF |
|  | 1540 FFFB |
|  | 2002 000A |
|  | 0000 000C |

And this is how I got there.

- (left): translated pseudoinstructions into MIPE

- (Frame on the right): figures out the appropriate format

- (Value10 row):Fill in the blanks with decimal values. Assuming that the PC runs on 16 bit addresses and    starts at 0000. We won’t bother converting the addresses to decimal to reconvert them in hex afterwards.

- (Binary): Converted the previous row in binary. Padded the upper bits with zeros to get appropriate 	bit allocation. Spaced out the data in 4 bites for easy conversion.

- (hex): Translated the binary MIPE in Hexadecimal, and there is the answer!!!

| Main |  | MIPEs | Format | Allocation and value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | li $10, 32 | Bytes | 6 | 5 | 5 | 16 |  |  |
|  |  | Equivalent | I | op | rs=source | rt=dest | Immediate |  |  |
|  |  | addi $10, $0, 32 | Value(10): | 8 | 0 | 10 | 32 |  |  |
|  |  |  | Binary: | 0010 00 | 00 000 | 0 1010 | 0000 0000 0010 0000 |  |  |
|  |  |  | Result | 200A 0020 |  |  |  |  |  |
|  |  | li, $6, 0 | Bytes (32) | 6 | 5 | 5 | 16 |  |  |
|  |  | Equivalent | I | op | rs=source | rt=dest | Immediate |  |  |
|  |  | addi $6, $0, 0 | Value(10) | 8 | 0 | 6 | 0 |  |  |
|  |  | 16 bit Address (in hex) | Bianry | 0010 00 | 00 000 | 0 0110 | 0000 0000 0000 0000 |  |  |
|  |  | 0 0 0 4 | Hex | 2006 0000 |  |  |  |  |  |

| Main10 |  | MIPEs | Format | Allocation and value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | andi, $12, $11, 1 | Bytes (32) | 6 | 5 | 5 | 16 |  |  |
|  |  |  | I | op | rs=source | rt=dest | Immediate |  |  |
|  |  |  | Value(10) | 12 | 11 | 12 | 1 |  |  |
|  |  |  | Bianry | 0011 00 | 01 011 | 0 1100 | 0000 0000 0000 0001 |  |  |
|  |  |  | Hex | 316C 0001 |  |  |  |  |  |
|  |  | beq $12, $0, main20 | Bytes (32) | 6 | 5 | 5 | 16 |  |  |
|  |  |  | I | op | rs=source | rt=dest | Immediate |  |  |
|  |  |  | Value(10) | 4 | 12 | 0 | Offset 2 lines = +2 |  |  |
|  |  |  | Bianry | 0001 00 | 01 100 | 0 0000 | 8 |  |  |
|  |  |  | Hex | 1180 0002 |  |  |  |  |  |
|  |  | addi $6, $6, 1 | Bytes (32) | 6 | 5 | 5 | 16 |  |  |
|  |  |  | I | op | rs=source | rt=dest | Immediate |  |  |
|  |  |  | Value(10) | 8 | 6 | 6 | 1 |  |  |
|  |  |  | Bianry | 0010 00 | 00 110 | 0 0110 | 0000 0000 0000 0001 |  |  |
|  |  |  | Hex | 20C6 0001 |  |  |  |  |  |

| Main 20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | srl $11, $11, 1 | Bytes (32) | 6 | 5 | 5 | 5 | 5 | 6 |
|  |  |  | R | op | rs=src1 | rt=src2 | rd=dest | shamt | funct |
|  |  |  | Value(10) | 0 | 0 | 11 | 11 | 1 | 2 |
|  |  |  | Bianry | 0000 00 | 00 000 | 0 1 011 | 0101 1 | 000 01 | 00 0010 |
|  |  |  | Hex | 000B 5842 |  |  |  |  |  |
|  |  | addi $10, $10, -1 | Bytes (32) | 6 | 5 | 5 | 16 |  |  |
|  |  |  | I | op | rs=source | rt=dest | Immediate |  |  |
|  |  |  | Value(10) | 8 | 10 | 10 | -1 |  |  |
|  |  |  | Bianry | 0010 00 | 01 010 | 0 1010 | 1111 1111 1111 1111 |  |  |
|  |  |  | Hex | 214A FFFF |  |  |  |  |  |
|  |  | bne $10, $0, main10 | Bytes (32) | 6 | 5 | 5 | 16 |  |  |
|  |  |  | I | op | rs=source | rt=dest | Immediate |  |  |
|  |  |  | Value(10) | 5 | 10 | 0 | Offset: -5 lines FFFB |  |  |
|  |  |  | Bianry | 0001 01 | 01 010 | 0 0000 | FFFB |  |  |
|  |  |  | Hex | 1540 FFFB |  |  |  |  |  |
|  |  | li $2, 10 | Bytes (32) | 6 | 5 | 5 | 16 |  |  |
|  |  | Equivalent | I | op | rs=source | rt=dest | Immediate |  |  |
|  |  | addi $2, $0, 10 | Value(10) | 8 | 0 | 2 | 10 |  |  |
|  |  |  | Bianry | 0010 00 | 00 000 | 0 0010 | 0000 0000 0000 1010 |  |  |
|  |  |  | Hex | 2002 000A |  |  |  |  |  |
|  |  | syscall | Bytes (32) | 6 | 20 |  |  |  | 6 |
|  |  |  | Special | 0 | 0 |  |  |  | 0xC |
|  |  |  | Value(10) | 0 | 0 |  |  |  | 12 |
|  |  |  | Bianry | 0000 00 | 00 0000 0000 0000 0000 00 |  |  |  | 00 1100 |
|  |  |  | Hex | 0000 000C |  |  |  |  |  |