ECSE211 (EML)			Experiment 1

By:

Frida Ceja		260 219 376

Simon Foucher	260 223 197

1- Basic Measurement

|  |  |  |  |  | Both +-5% |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E (V) | V1 (V) | V2 (V) | I (mA) | R1 (kΩ) | R2 (kΩ) |
| Theoretical | 9.50 | 6.06 | 3.44 | 0.505 | 12.00 | 6.80 |
| As Measured (+-0.001) | 9.491 | 6.086 | 3.400 | 0.502 | 12.099 | 6.754 |
| Difference | 0.01 | -0.03 | 0.04 | 0.003 | -0.10 | 0.05 |

Table 1: Measured vs. Theoretical values

Calculations using Ohm’s law:

R1  =   6.086 V     =     12 123.5 Ω

502 mA

R2  =   3.400 V     =     6 773 Ω

502 mA

Observation: The values measured are really similar to the theoretical values we were anticipating. We can conclude that Ohm’s law is indeed an accurate relation and that our measuring equipment offers great exactitude at good precision.

2- Loading Effect

(a) Ammeter

Actual values used:

E = 1.0006V

R1 = 1 491.9 Ω

R2 = 476 Ω

Theoretical data:

V1 = R1/(R1+R2)

V2 = R2/(R1+R2)

I = E/(R1+R2)

|  | Short Circuit |  |  |  |
| --- | --- | --- | --- | --- |
|  | Measured | Precision (+-) | Theoretical | Difference |
| V1 (V) | 0.7576 | 0.0001 | 0.7581 | 0.0005 |
| V2 (V) | 0.2417 | 0.0001 | 0.2419 | 0.0002 |
| Vm (V) | 0 | 0.0001 | 0 | 0.0000 |
| I Samson (μA) | 0 | 1 | 482 | N/A |
| I Real (μA) | 506 | 0.1 | 508.5 | 2.5 |

Table 2a.1: Values using short circuit mode

|  | 500 μA Scale |  |  |  |
| --- | --- | --- | --- | --- |
|  | Measured | Precision (+-) | Theoretical | Difference |
| V1 (V) | 0.6411 | 0.0001 | 0.7581 | 0.1170 |
| V2 (V) | 0.2044 | 0.0001 | 0.2419 | 0.0375 |
| Vm (V) | 0.154 | 0.0001 | 0 | -0.1540 |
| I Samson (μA) | 440 | 1 | 482 | 42 |
| I Real (μA) | 428 | 0.1 | 508.5 | 80.5 |

Table 2a.2: Values using 500 scale.

|  | 1000 μA Scale |  |  |  |
| --- | --- | --- | --- | --- |
|  | Measured | Precision (+-) | Theoretical | Difference |
| V1 (V) | 0.6911 | 0.0001 | 0.7581 | 0.0670 |
| V2 (V) | 0.2203 | 0.0001 | 0.2419 | 0.0216 |
| Vm (V) | 0.088 | 0.0001 | 0 | -0.0880 |
| I Samson (μA) | 420 | 1 | 482 | 62 |
| I Real (μA) | 462 | 0.1 | 508.5 | 46.5 |

Table 2a.3 Values using 1000 scale.

In short circuit mode, the voltage drop across the meter is 0 and its presence only affects the circuit voltages by a few microvolts. The multimeter doesn’t present much change in the current because its loading effects are small.

The microammeter’s presence is the most obvious on the 500 scale, where we observed the largest voltage drop across the meter, as well as the largest difference between theoretical voltages and currents and real values. As we moved to the 1000 scale, the divergence observed was reduced, due to the fact that on a bigger scale, there is a smaller internal resistance.

In general, presence of the microammeter loads the circuit by decreasing the current because the resistance is increased, and it reduces voltage drops across present resistors because now the voltages have to divide among more resistances.

Theoretical resistance values based on our IV measurements:

For short circuit:

R1= V1/I= (0.7576 V/.000506)= 1497.23 Ω

R2= V2/I= (0.2417/.000506)= 477.67 Ω

For 500 μA:

R1= V1/I= (0.6411V/.000428)= 1497.89 Ω

R2= V2/I= (0.2044/.000428)= 477.57 Ω

For 1000 μA:

R1= V1/I= (0.6911V/.000462)= 1495.88 Ω

R2= V2/I= (0.2203/.000462)= 476.83 Ω

Internal resistance of the meter on a 500μA scale:

R = V/I = 154mV / 0.428mA = 359.8 Ω

Internal resistance of the meter on a 1000μA scale:

R = V/I = 88mV / 0.462mA = 190.48 Ω

More accurate model:

Figure 1: More accurate model with ammeter

New theoretical values on a 500μA scale:

I = E / Rt = 1.0006V / (1491.9Ω + 476Ω + 359.8 Ω) = 429.9 μA

V1 = E(R1 / (R1+R2+RI)

V1 = 1.0006V (1 491.9Ω  / (1491.9Ω + 476Ω + 359.8 Ω))

V1 = 0.6413V

V2 = E(R2 / (R1+R2+RI)

V2 = 1.0006V (476Ω  / (1491.9Ω + 476Ω + 359.8 Ω))

V2 = 0.2046V

|  | 500 μA Scale |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Measured | Theoretical | Difference | Theoretical adj | Difference |
| V1 (V) | 0.6411 | 0.7581 | 0.1170 | 0.6413 | 0.0002 |
| V2 (V) | 0.2044 | 0.2419 | 0.0375 | 0.2046 | 0.0002 |
| I (μA) | 428 | 527.5 | 99.5000 | 429.9 | 1.9000 |

Table 2a.4: Values using new model 500 scale

We can observe that the gap between the adjusted theoretical values and the measured values has been significantly reduced.

New theoretical values on a 1000μA:

I = E / Rt = 1.0006V / (1491.9Ω + 476Ω + 190.48 Ω) = 463.6 μA

V1 = E(R1 / (R1+R2+RI)

V1 = 1.0006V (1 491.9Ω  / (1491.9Ω + 476Ω + 190.5 Ω))

V1 = 0.6916V

V2 = E(R2 / (R1+R2+RI)

V2 = 1.0006V (476Ω  / (1491.9Ω + 476Ω + 190.5 Ω))

V2 = 0.2207V

|  | 1000 μA Scale |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Measured | Theoretical | Difference | Theoretical adj | Difference |
| V1 (V) | 0.6911 | 0.7581 | 0.0670 | 0.6916 | 0.0005 |
| V2 (V) | 0.2203 | 0.2419 | 0.0216 | 0.2207 | 0.0004 |
| I (μA) | 462 | 527.5 | 65.5000 | 463.6 | 1.6000 |

Table 2a.5: Values using new model 1000 scale

Here we can observe an even smaller gap between values.

Relation derived:

I = E / Rt

I = E / (R1 + R2 + Ra)

Since (R1+R2)/(R1+R2) = 1

I = E/ (R1 + R2 + [Ra x (R1+R2) / (R1+R2)]

Factoring out R1+R2

I = E / [ (R1+R2) (1+Ra/(R1+R2))]

Since the theoretical circuit value of current should be  I = E/(R1+R2), by adding a meter, we are dividing E by an additional factor of Ra/(R1+R2). When Ra is close to 0, the circuit will behave normally. As we increase the internal resistance of the meter with respect to a fixed R1+R2, we are increasing the denominator of the above relation, so under a fixed E, I will be reduced.

We can conclude that the loading effect is proportional to the ratio Ra/Rceq, where Ra is the internal resistance of the meter and Rceq is the Norton equivalent resistance of the circuit excluding the meter. The ammeter will significantly load the circuit when its internal resistance has a magnitude similar or greater than the resistances in the circuit.

b) Voltmeter

Resistance M using Ohm’s law

We measured directly the voltage across the terminals of the ammeter and we took the current of the ammeter.

V= 41.41 mV

I= 40 μA

Rm= V/I= (0.004141/0.000040)= 1035.25 Ω

Rs= R variable + 82 kΩ

Rvariable (measured)= 14.498 kΩ

# Rs=96.498 kΩ

Rs theoretical= 98.964 kΩ

Rs – Rs theoretical= 2.466 kΩ

| Standard meter (+- .001 V) | Our meter (+- .1 V) |
| --- | --- |
| 9.999 | 10.0 |
| 8.985 | 9.0 |
| 7.983 | 8.0 |
| 7.016 | 7.0 |
| 6.009 | 6.0 |
| 5.017 | 5.0 |
| 3.987 | 4.0 |
| 3.011 | 3.0 |
| 1.998 | 2.0 |
| .993 | 1.0 |

Table 2b.1: Standard voltmeter vs. ours

Figure 2:Voltages measured as a function of the scaled subdivisions

If by “sensitivity” of out meter you mean the smallest fluctuation in voltage that will cause a change in meter reading (i.e. precision), it is 0.1V. As far as the exactitude of our measurements goes (scaled by the Flook meter), it is fair to say that our meter gives a pretty accurate reading, because given the same amount of significant figures on the Flook meter, all values are identical.

If we change the multiplier resistor in order to get a 100V scale, the precision of the meter will be reduced by a factor of 10. This is due to the fact that the display of the meter remains the same size, therefore fitting 10 times more values will effectively decrease the precision by 10 times. The exactitude of the measurements shouldn’t be affected, because there will be the same ammout of current flowing through the coil that makes the needle move, therefore the deflection caused by the emf should remain constant.

|  | Theoretical value (V) | Our voltmeter (+-.1V) | Multimeter (+-.001V) |
| --- | --- | --- | --- |
| ## V1 | ## 6.5 | 2.8 | 6.370 |
| V2 | 3 | 1.3 | 3.008 |

Table 2b.2: Voltmeters loading effects

The measurements obtained with the multimeter are very close to the theoretical values.  The voltages obtained with our meter are smaller than the real ones because our voltmeter draws current from the circuit. The voltage is the resistance times the current. As the current passing through the resistors is decreased, the voltage measured is smaller.

MORE ACCURATE MODEL

Figure 3: More accurate model with voltmeter

CHANGES IN CURRENT

Currents measured by the multimeter:

| Without our voltmeter connected | .017 +- .001 mA |
| --- | --- |
| With our voltmeter connected to measure V1 | .037+- .001 mA |
| With our voltmeter connected to measure V2 | .022+- .001 mA |

Table 2b.3: Changes in current

We notice that when our voltmeter is connected the current in the circuit increases. This is because the resistance in our meter is added in parallel with one of the resistances. This decreases the total resistance the circuit sees, therefore the current increases.

From the model,

V2 = E * (R2||Rm) / (R1 + R2||Rm)

(R2||Rm) = R2Rm/(R2+Rm)

V2 = E * [R2Rm/(R2+Rm)] / [R1 + (R2Rm/(R2+Rm)]

V2 = E * [R2Rm/(R2+Rm)] / [(R1R2 + R1Rm + R2Rm)/(R2+Rm)]

V2 = E * [R2Rm] / [(R1R2 + R1Rm + R2Rm)]

V2 = E * R2 / (R1R2/Rm + R1 + R2)

Dividing numerator by (R1+R2) and denominator by (R1+R2)

V2 = [(E * R2) / (R1+R2)] / [(R1R2)/(Rm(R1+R2)) + 1]

Separating E and R2

V2 = R2 / (R1+R2) * E / [1 + R1R2/(R1+R2)Rm]

When the voltmeter’s internal resistance is smaller or similar to the ones in the circuit, it will significantly load the circuit.

3- Ammeter Implementation

(i) R= 2.2 KΩ

Rshunt= 47Ω+ variable resistance= 47 Ω+ 40.7Ω= 87.7Ω

Rshunt theoretical= (.111)Rm=(.111)(1035.25Ω)=114.9Ω

Rshunt theoretical-Rshunt= 27.2 Ω

We can see the difference between the theoretical and the measured values is not too big.

| Standard meter (+- .001 Ma) | Our meter (+- .1 mA) |
| --- | --- |
| 5.020 | 5.0 |
| 4.509 | 4.5 |
| 4.007 | 4.0 |
| 3.497 | 3.5 |
| 3.006 | 3.0 |
| 2.497 | 2.5 |
| 2.006 | 2.0 |
| 1.499 | 1.5 |
| 1.004 | 1.0 |
| .500 | .5 |

Table 3.1: Standard ammeter vs. ours

Figure 4: Currents measured as a function of the scaled subdivisions

(ii) With our meter  I= .5 mA

With multimeter  I= .503 mA

Theoretical value  I= .505 mA

We can see both values are very close to the values calculated in the preparation. Of course, the value obtained with the multimeter is closer to the expected value.

MORE ACCURATE MODEL:

Figure 5: Model with measured resistances

The total resistance of the ammeter is:

(Rshunt)(Rm)/(Rshunt + Rm)= 80.85 Ω

None of the meters cause significant loading. The reason for which our meter doesn’t cause significant loading is that its resistance is small in comparison to the circuit’s resistance.

With our meter plugged:

V1= 6.078 +- .001V

V2= 3.395 +- .001V

V1= E(R1)/(R1+R2+Rammeter)= 6.037 V

V2= E(R2)/(R1+R2+Rammeter)= 3.420 V