Assignment 4

Due October 10 (Fri), 2008.

=====================Part 1 (no submission is required)=========================

Practice makes perfect. Do and understand all exercises in Chapter 4 of Benoit Boulet’s book.

==================Part 2 (Handwritten and submission are required)==================

4.1 A periodic rectangular wave as shown below has a frequency of 5kHz, pulse width =20 s, amplitude E=10 V.  Derive the amplitudes of DC component, fundamental harmonic component and Nth harmonic component.

-0.5    0    0.5

4.2 Exercise 4.6 of Boulet’s book.

4.3 Exercise 4.8 of Boulet’s book.

Note: to do problems (b) and (c), you need to study Matlab files linespectrum.m and Fourierseries.m on the CD \Chapter4 and create your m file.

- In the Matlab command window, you type: help eval  (and then press the return key), and learn the Matlab built-in function “eval”.

- Replace the string ‘=-j*A/(k*pi)’ (in linespectrum.m), which is the FS coefficient value for the signal in Figure 4.3, with the FS coefficient value for the signal in Figure 4.32

- Study lines 93-109 of Fourierseries.m and learn how to sum a number of harmonic components .

4.5 Exercise 4.10 of Boulet’s book.

4.6 Let x(t) and y(t) both be continuous-time periodic signal having period T and with Fourier series representations given by

x(t)=k=−∞∞akejkw0t,y(t)=k=−∞∞bkejkw0t

- show that the Fourier series coefficients of the signal z(t)

z(t)=x(t)y(t)=k=−∞∞ckejkw0t where  ck=n=−∞∞anbk−n

- Suppose that y(t)=x(t). Express the bk in terms of ak, and use the result of part (a) to prove Paseval’s relation for periodic signals:

1T|x(t)|2dt=k=−∞∞|ak|2