McGill University

Department of Electrical and Computer  Engineering

Course: ECSE-323 Digital Systems Design			Fall 2008

Assignment #7

With Solutions

TOPIC: Finite State Machines I

1.-	A sequential network has one input  X  and one output  Z . Draw a Moore state transition diagram for the following case:

The output  Z = 1  if and only if the total number of  0’s  received is even, and  Z = 0  otherwise. (Assume  0  number of  0’s  received to be even)

a)	Draw a state transition diagram for the circuit using few numbers of states.

b)	Draw the state transition table

c) 	Draw a circuit producing the state transition table found in b) and using  T-type edge-trigger flip-flops.

____________________________________________________________

ANSWER

a)

b)

| State Q | Next State |  | Z | T |  |
| --- | --- | --- | --- | --- | --- |
|  | X = 0 | X = 1 |  | X = 0 | X = 1 |
| 0  even | 1 | 0 | 1 | 1 | 0 |
| 1  odd | 0 | 1 | 0 | 1 | 0 |

______

Q+ = X  Q                   Z = Q               T =X

c)

________________________________________________________________

2.-	A Mealy-type FSM sequential circuit is required to check four bits of data.

The data arrive serially and the output from the circuit is required to be equal to

1 only if the total number of  1’s received is even.

a)	Draw a sate transition diagram for the circuit (7 states are sufficient)

b)	Draw a sate transition table (Assume that  0  number of 1’s is even)

________________________________________________________________

ANSWER

b)

| Present state | Next state |  | Output Z |  |
| --- | --- | --- | --- | --- |
|  | I = 0 | I = 1 | I = 0 | I = 1 |
| start | 1 | 2 | 0 | 0 |
| 1 | 3 | 4 | 0 | 0 |
| 2 | 4 | 3 | 0 | 0 |
| 3 | even | even | 0 | 0 |
| 4 | odd | odd | 0 | 0 |
| even | start | start | 1 | 0 |
| odd | start | start | 0 | 1 |

_______________________________________________________________

3.-	Design a serial  2’s complementer. The algorithm used is the following:

The bits of the number  N  are examined from right to left, copying the bits until the first  1-bit  has been copied and then complementing the remaining bits

Example

Let N be       0  0  1  1  1  0  0  1  1  1  0  0

Bits complemented                          bits copied

Result          1  1  0  0  0  1  1  0  0  1  0  0

The data bits are supplied  to the circuit sequentially on input  x  beginning with the least significant bit. The circuit output  Z  is the correct data bit.

a)	Produce the state transition diagram of the Mealy-type FSM representing the serial 2’s complementer.

b)	Produce the corresponding circuit using D-type edge-trigger flip-flops and NAND gates

_____________________________________________________________

ANSWER

a)

b)

| Present state |  | Next state |  | Output Z |  |
| --- | --- | --- | --- | --- | --- |
| state | code | X = 0 | X = 1 | X = 0 | X = 1 |
| S0 | 0 | 0 | 1 | 0 | 1 |
| S1 | 1 | 1 | 1 | 1 | 0 |

Q+ = Q + X ;   D = Q + X    Z = Q  X

c)

________________________________________________________________

4.- 	Consider the following synchronous sequential circuit

a)	Give the state transition diagram of the circuit

b)	Is this a Mealy or a Moore type Finite State Machine

c)	What is the output sequence generated by the input sequence

x = 0 1 1 0 1 1 1 0 1 0, assuming starting state  00.

Note:  For part c) read the input sequence from left to right, meaning that  x  assumes the values 0,1,1,0,1,1,1,0,1,0 at times t1,t2,t3,t4,t5,t6,t7,t8,t9,t10 respectively.

_____________________________________________________________

ANSWER

a)  Z1 =  (x  y1)  ;  Z2 = (x  y1) y2

y1+ = Dy1 =(x  y1)                  y2+ = Dy2 = (x  y1) y2

| Present state | Next State |  | Outputs Z1 Z2 |  |
| --- | --- | --- | --- | --- |
| y1 y2 | X = 0 | X = 1 | X = 0 | X = 1 |
| 0 0 | 0 0 | 1 1 | 0 0 | 1 1 |
| 0 1 | 0 0 | 1 0 | 0 1 | 1 0 |
| 1 0 | 1 1 | 0 1 | 1 1 | 0 1 |
| 1 1 | 1 0 | 0 0 | 1 0 | 0 0 |

b)  It is a Mealy-type  FSM because outputs depend on present sate and input  X

c)  Z1 : 0  1  0  0   1  0  1  1  0  0

Z2 : 0  1  0  0   1  0  1  0  1  0

_______________________________________________________________

5.-	Design a Moore-type FSM sequence recognizer that detects the presence of  3  or more consecutive  1’s  at its input line  X . Overlapping sequences are to be recognized. The circuit has one input  X  and one output  Z.

Example:

X:  	0  1  1  1  1  0  0  1  1  1  0  0…

Z:       0  0  0  1  1  0  0  0  0  1  0  0…

a)  Produce the state transition diagram (4 states are sufficient)

b)  Give the excitation equations of the D-type edge-trigger flip=flops and draw the complete circuit using AND, OR gates.

_____________________________________________________________________________

ANSWER

a)

b)

| Present |  | Next state |  | Output |
| --- | --- | --- | --- | --- |
| state | code | X = 0 | X = 1 | Z |
| 1 | 0 0 | 0 0 | 0 1 | 0 |
| 2 | 0 1 | 0 0 | 1 0 | 0 |
| 3 | 1 0 | 0 0 | 1 1 | 0 |
| 4 | 1 1 | 0 0 | 1 1 | 1 |

y1  y2                           y1+ y2+

y1+ = y1 X + y2 X

y2+ = y1 X +y2 X

Z = y1 y2

c)

********************END OF ASSIGNMENT #7 (with solutions)*********************