McGill University

Department of Electrical and Computer  Engineering

Course: ECSE-323 Digital Systems Design			FALL 2008

Assignment #9

With Solutions

TOPIC: Finite State Machines II

Optimization of Finite State Machines

Analysis of Synchronous Sequential Circuits

__________________________________________________________________

1.-

Using the partitioning method reduce the following state transition table so that it contains the minimum number of states.

PRESET       NEXT STATE           OUTPUT

STATE          Present input            Present input

00	01	10	00	01	10

A	E	C	B	1	0	1

B	C	F	E	0	1	1

C	B	A	D	1	0	1

D	C	F	E	0	1	1

E 	A	F	B	0	1	1

F	C	E	C	0	1	1

a) Give the first partition

a) Give the second partition

c) Give the third partition and the reduced transition table

_______________________________________________________________________________________

ANSWER

a) First partition different outputs (A C) (B D E F)

b) Second partition  and final (A C) (B D E) (F)

c) Third  partition is the same as second thus the process terminates

The reduced table is shown below

| Present sate | Next state Present input 00 01 10 | Output Present input 00 01 10 |
| --- | --- | --- |
| A | B A B | 1 0 1 |
| B | A F B | 0 1 1 |
| F | A B A | 0 1 1 |

2.-  Using the partitioning method reduce the following state transition table so

that contains the minimum number of states

| Present State | Next State Present Inputs X = 0 X = 1 | Output Present Inputs X = 0 X = 1 |
| --- | --- | --- |
| A | B C | 0 0 |
| B | D E | 0 0 |
| C | F G | 0 0 |
| D | H I | 0 0 |
| E | J K | 0 0 |
| F | L N | 0 0 |
| G | N P | 0 0 |
| H | A A | 0 0 |
| I | A A | 0 0 |
| J | A A | 0 1 |
| K | A A | 0 0 |
| L | A A | 0 1 |
| M | A A | 0 0 |
| N | A A | 0 0 |
| P | A A | 0 0 |

a) Give the first partition

b) Give the second partition

c) Give the final reduced transition table

_____________________________________________________________

ANSWER

a)First partition on different outputs  (A B C D E F G H M N P) (J L)

b) Second and following partitions (A B C D E F G) (H I K M N P) (J L)

(A) (B) (C) (D G) (E F) (H I K M N P) (J L)

c)

| Present sate | Next State | Output |
| --- | --- | --- |
|  | X = 0 X = 1 | X = 0 X = 1 |
| A | B C | 0 0 |
| B | D E | 0 0 |
| C | E D | 0 0 |
| D | H H | 0 0 |
| F | J H | 0 0 |
| H | A A | 0 0 |
| I | A A | 0 1 |

3.-

Consider the following circuit

a) Give the transition diagram of the circuit

b) Is this circuit a Moore or a Mealy-type FSM?

c)Give the state transition diagram of the Mealy-

type equivalent, or vice versa, if it is a Mealy-type, give the state transition diagram of the

Moore-type equivalent.

_____________________________________________________________

ANSWER

a)

A+ = DA = x B

B+ = DB = x + A                  Z = A  B

| P.S. | X = 0 | X = 1 | Z |
| --- | --- | --- | --- |
| S1: 00 | 1 0 | 0 1 | 0 |
| S2: 01 | 0 0 | 1 1 | 1 |
| S3: 10 | 1 1 | 0 1 | 1 |
| S4: 11 | 0 1 | 1 1 | 0 |
| A B | A+ B+ |  |  |

c) The circuit represents a Moore-type FSM because the output depends only of the present state of the flip-flops. The state transition diagram of the equivalent Mealy-type is

_____________________________________________________________

4.-

Consider the following synchronous sequential circuit

a)  Give the excitation table for the circuit

b)  Give the state transition table of the circuit

c)  Give the state transition diagram of the circuit

_____________________________________________________________

ANSWER

a)

J1 = y2  X                          J2 = X                                   Z = X y1 y2

K1 =X                              K2 =y1 +X

|  |  | J1 K1 Excitation X = 0 X = 1 |  |  |  | J2 K2 Excitation X = 0 X = 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P. S y1 y2 |  |  |  |  |  |  |  |  | Output Z X = 0 X = 1 |
| 0 0 |  | 0 1 |  | 0 0 |  | 0 1 | 1 1 |  | 0 0 |
| 0 1 |  | 0 1 |  | 1 0 |  | 0 1 | 1 1 |  | 0 0 |
| 1 0 |  | 0 1 |  | 0 0 |  | 0 1 | 1 0 |  | 0 0 |
| 1 1 |  | 0 1 |  | 1 0 |  | 0 1 | 1 0 |  | 0 1 |

| JK | Q+ |
| --- | --- |
| 00 | Q |
| 10 | 1 |
| 01 | 0 |
| 11 | Q |

b) From the excitation values and the behaviour of the JK-flipflop we obtain the assigned

state table

| P. S | Next State y1+ y2+ |  |  | Output Z |  |
| --- | --- | --- | --- | --- | --- |
| y1 y2 | X = 0 X = 1 |  |  | X = 0 X = 1 |  |
| 0 0 | 0 0 |  | 0 1 | 0 0 |  |
| 0 1 | 0 0 |  | 1 0 | 0 0 |  |
| 1 0 | 0 0 |  | 1 1 | 0 0 |  |
| 1 1 | 0 0 |  | 1 1 | 0 1 |  |

| P. S | Next State y1+ y2+ |  | Output Z |
| --- | --- | --- | --- |
| y1 y2 | X = 0 X = 1 |  | X = 0 X = 1 |
| A | A | B | 0 0 |
| B | A | C | 0 0 |
| C | A | D | 0 0 |
| D | A | D | 0 1 |

Assigned state table                                                       Generic state table

c) The Mealy-type state transition diagram of the given circuit is the following

********************END OF ASSIGNMENT #9 (with solutions)*********************