Question 1 : Sequential Circuits (10 points)

A U-V flip-flop behaves according to the following description

If  U-V is  00  the output Q complements

If  U-V is  01  the output Q is set to  1

If  U-V is  10  the output Q is reset to  0

If  U-V is  11  the output Q does not change.

(5 points)	a)	Complete the following table and use don’t care values where possible

| Q | Q+ | U | V |
| --- | --- | --- | --- |
| 0 | 0 |  |  |
| 0 | 1 |  |  |
| 1 | 0 |  |  |
| 1 | 1 |  |  |

(5 points) 	b) 	Draw the circuit of a J-K flip-flop using a U-V flip-flop  and fewest

number of gates.

| ANSWER: a) Q | Q+ | U | V | J | K |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | x | 0 | x |
| 0 | 1 | 0 | x | 1 | x |
| 1 | 0 | x | 0 | x | 1 |
| 1 | 1 | x | 1 | x | 0 |

| Q | Q+ | U | V |
| --- | --- | --- | --- |
| 0 | 0 | 1 | x |
| 0 | 1 | 0 | x |
| 1 | 0 | x | 0 |
| 1 | 1 | x | 1 |

| JK Q 00 01 11 10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 0 | x | x | 0 | x |
| 1 | x | x | 0 | 1 |

| JK Q 00 01 11 10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 0 | x |
| 1 | x | x | 0 | 1 |

U = J +K                                                                               V = K

There are many solutions depending on the values given to the don’t cares. One solution is the following:

# Question 2 : Registers (10 points)

# Draw the circuit of a 4-bit multimode parallel-load register which produces the following functions depending on the values of the mode selection variables s1, s2

| # Mode selection # s1, s2 | # Function to be performed for i = 1,2,3,4 |
| --- | --- |
| # 0 0 | # Ri  Ii |
| # 0 1 | # Ri  EX-OR (Ri,Ii) |
| # 1 0 | # Ri  NAND( Ri, Ii ) |
| # 1 1 | # R  NOR (Ri, Ii) |

# where  {R4, R3, R2, R1} is the 4-bit register and  {I4, I3, I2, I1} the 4 input lines.

# Use  4 x 1 multiplexers for mode selection and the fewest number of gates possible.

# Marking Scheme: 2 point for each correct mode and 2 points for a neat readable drawing.

# ________________________________________________________________

ANSWER

The complete circuit is composed of four identical logic circuit slices, each one of them contains one flip-flop, Ri, of the register {R4, R3, R2, R1}. Following we show only one of the four logic circuit slices. (i = 1,2,3,4).

Question 3 : Analysis of Synchronous Sequential Circuits  (15 points)

Given the following circuit

(5 points)	a)	Produce the excitation equations of the flip-flops

(5 points)	b)	Produce the state-transition-diagram of the circuit

(2 points)	c)	Is this circuit a MOORE or a MEALY type Finite State Machine?

Reason your answer.

(3 points)	d)	If tp = 10 ns, tg = 5 ns, tsu = 5 ns, what is the fastest clock rate for the circuit to function

properly?  (tp = flip-flop propagation delay, tg = gate propagation delay and  tsu = flipflop                 set-up time).

______________________________________________________________________________

a)

| Jy1= x (y1 + y2) Ky1= x y1 y2 | Jy2= y1y2 Ky2= y1 +y2 |
| --- | --- |

b)

| Present State | Next state : y1+y2+ |  | Output Z = x y1 y2 |
| --- | --- | --- | --- |
| y1 y2 | x = 0 x =1 | x = 0 x = 1 |  |
| 0 0 | 0 0 0 0 | 0 0 |  |
| 0 1 | 0 0 1 0 | 0 0 |  |
| 1 0 | 1 1 1 1 | 0 0 |  |
| 1 1 | 0 1 1 1 | 0 1 |  |

c)

d)  f = 1/(tp + 3tc + tsu ) = ( 1/30) x 109 = 0.033 x 109 = 33 MHz

Question 4 : Counter Design(15 points)

# Using  the  74X163  4-bit programmable counter, with synchronous CLEAR and LOAD inputs, whose block diagram is shown below, and the fewest number of  gates possible,  draw a circuit of a counter having 4 different counting sequences depending on the values of two mode selection variables s1 , s2 . The 4 counting sequences of the counter are the following

| # Mode selection, s1 , s2 | # Counting sequence of the counter |
| --- | --- |
| # 0 0 | # 0 to 15, (0,1,2,..,15,0,1,2,…15,0,1,2,..) |
| # 0 1 | # 0 to 7, (0,1,2,…,7,0,1,2,…,7,0,1,2…) |
| # 1 0 | # 0 to 9, (0,1,2,…,9,0,1,2,…,9,0,1,2…) |
| # 11 | # 0 to 12, (0,1,2,..,12,0,1,2,…12,0,1,2…) |

# Use  4  4x1 multiplexers for counting mode selection. Assume that the counter is self-starting, meaning that the counter will always fall into the correct selected counting sequence after certain number of counts have elapsed.

# Marking scheme:  	3 points for each correct counting mode and 3 additional points for a neat and readable drawing of the circuit.

NOTE: CLR (CLEAR) and LD (LOAD) are synchronous inputs, meaning, that they affect the counter on the following rising edge of the clock. ENP and ENT should be  1  for the counter to be ENABLED. QD is the most significant bit of the counter. RCO is the Ripple-Carry-Output signal.

__________________________________________________________________________________________

Question 5 : Finite State Machine Design (10 points)

Design a Mealy type FSM, having one input  X  and one output  Z . The output Z is to be 1 when the sequence 101 been received at the input X  and  0  otherwise. Assume overlapping sequences.

Example:  X = 0  0  1  0  1  1  1  0  0  0  1  0  1  0  1  0  0  1  0  1  0  0……

Z = 0  0  0  0  1  0  0  0  0  0  0  0  1  0  1  0  0  0  0  1  0  0……

(4 points)	a)	Produce a state-transition diagram of this FSM and make it self-starting.

(3 points)	b)	Produce the state-transition-table showing to with the D input excitations                                                                       of the flip-flops.

(3 points)	c)	Draw the complete circuit.

a)

b)         Dy1=y1+  ;   Dy=y2+

| Present State | Next state : y1+y2+ |  | Output Z = x y1 y2 |
| --- | --- | --- | --- |
| y1 y2 | x = 0 x = 1 | x = 0 x=1 |  |
| A : 0 0 | 0 0 0 1 | 0 0 |  |
| B : 0 1 | 1 0 0 0 | 0 0 |  |
| C : 1 0 | 0 0 0 1 | 0 1 |  |
| D : 1 1 | 0 0 0 0 | 0 0 |  |

c)	Dy1=y1+  = xy1  y2 ;   Dy=y2+ =  xy2

Question 6 : VHDL (15 points)

Write the architecture block for a VHDL description of a complex 7-bit shift register, with the following features:

- asynchronous clear

- synchronous parallel load (with enable)

- synchronous serial shift in with selectable shift direction (left or right) (with enable)

- a 3-bit count output which indicates how many '1'-bits are in the register

The parallel load should take precedence over the serial shift. There is only one shift_in line and one shift_out line, which are used for both shift directions.

- (3 marks) Draw the symbol for the register, showing all inputs, outputs, and control signals, as well as the number of bits for each signal.

Data[6..0]          Q[6..0]

S_In                   S_Out

Dir

LD_En       Count[2..0]

AClr

Clock

(continued on next page…)

Question 6 : VHDL (continued…)

- (12 marks) Write the architecture block for the VHDL description of the register.

architecture mt2 of shiftreg is

begin

process(clock, aclr)

begin

if aclr = '1' then

count <= "000";

Q <= "0000000";

elsif clock='1' and clock'EVENT then

if LD_EN = '1' then  -- parallel load

Q <= Data;

count <= Data(6)+Data(5)+Data(4)+

Data(3)+Data(2)+Data(1)+Data(0);

elsif dir='1' then  -- right shift

Q <= S_In & Q(6 downto 1);

S_Out <= Q(0);

count <= count + S_In – Q(0);

else   -- left shift

Q <= Q(5 downto 0) & S_In;

S_Out <= Q(6);

count <= count + S_In – Q(6);

end if;

end if;

end process;

end mt2;