McGill University

Department of Electrical and Computer  Engineering

Course: ECSE 323  -Digital Systems Design				Winter 2008

ASSIGNMENT #1

(with Solutions)

# PROBLEMS ARE FROM BROWN’S TEXTBOOK 2nd. ED.

# TOPIC:  BOOLEAN LOGIC THEORY

1.-      Problem 2.3 page 67

___________________________________________________________________________

ANSWER

Manipulate the left hand side as follows:

xy + yz +x z  =  xy + (x +x)yz + xz

=  xy + xyz +xyz +xz

= xy(1 + z) +x(y + 1)z

= xy . 1 +x . 1. z

= xy +xz.

__________________________________________________________________________

2.-	Problem 2.36 page 71

___________________________________________________________________________

ANSWER

a)

| Identifier | x1 x0 y1 y0 | f |
| --- | --- | --- |
| 0 | 0 0 0 0 | 1 |
| 1 | 0 0 0 1 | 0 |
| 2 | 0 0 1 0 | 0 |
| 3 | 0 0 1 1 | 0 |
| 4 | 0 1 0 0 | 1 |
| 5 | 0 1 0 1 | 1 |
| 6 | 0 1 1 0 | 0 |
| 7 | 0 1 1 1 | 0 |
| 8 | 1 0 0 0 | 1 |
| 9 | 1 0 0 1 | 1 |
| 10 | 1 0 1 0 | 1 |
| 11 | 1 0 1 1 | 0 |
| 12 | 1 1 0 0 | 1 |
| 13 | 1 1 0 1 | 1 |
| 14 | 1 1 1 0 | 1 |
| 15 | 1 1 1 1 | 1 |

b)  The canonical SOP expression is

f  =  x1 x0 y1 y0 + x1  x0 y1 y0 + x1  x0 y1  y0 + x1 x0 y1 y0 + x1 x0 y1  y0 +

+  x1  x0  y1 y0 + x1  x0 y1  y0 +  x1  x0 y1  y0 +  x1  x0  y1 y0 + x1  x0  y1  y0 .

c)  The simplest SOP expression is

f  =  x1  x0  + y1 y0 + x1y0 + x0 y1.

___________________________________________________________________________

3.-        A function  f(A,B)  is self-dual if the dual of  f,  fD,  is such that  f = fD , where

fD = `f (`A,`B ). Give all self-dual functions of two variables.

__________________________________________________________________________

ANSWER

All the functions,   f i ,  0 < i < 15 , of two variables are shown below

| x1x0 | f 0 | f 1 | f 2 | f 3 | f 4 | f 5 | f 6 | f 7 | f 8 | f 9 | f 10 | f 11 | f 12 | f 13 | f 14 | f 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 0 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 1 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| 1 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

All the dual functions of two variables are obtained by changing all  0’s into 1’s and vice versa in the above table, including the values  of the logic space. We obtain the following table

| x1x0 | f0d | f1d | f2d | f3d | f4d | f5d | f6d | f7d | f8d | f9d | f10d | f11d | f12d | f13d | f14d | f15d |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 1 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 0 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 0 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Comparing the truth tables of each function and its dual, we identify the self-dual functions.

They are:  f 3 , f 5 , f 10 , f 12 . ___________________________________________________________________________

4.-	Problem 4.23 page 238

__________________________________________________________________________ ANSWER

Grouping the minters                                              First and only reduction

by number of  1’s

x1  x2  x3  x4

| 0 | 0 0 0 0 |  |
| --- | --- | --- |
| 2 4 8 | 0 0 1 0 0 1 0 0 1 0 0 0 | √ √ √ √ √ √ √ |
| 5 9 | 0 1 0 1 1 0 0 1 |  |
| 7 | 0 1 1 1 |  |
| 15 | 1 1 1 1 |  |

| 0,2 | 0 0 - 0 |
| --- | --- |
| 0,4 | 0 - 0 0 |
| 0,8 | - 0 0 0 |
| 4.5 | 0 1 0 - |
| 8,9 | 1 0 0 - |
| 5,7 | 0 1 - 1 |
| 7,15 | - 1 1 1 |

COVERING TABLE

| Prime implicants | MINTERMS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 2 | 4 | 5 | 7 | 8 | 9 | 15 |
| P1 = 0 0 - 0 | √ | √ |  |  |  |  |  |  |
| P2 = 0 - 0 0 | √ |  | √ |  |  |  |  |  |
| P3 = - 0 0 0 | √ |  |  |  |  | √ |  |  |
| P4 = 0 1 0 - |  |  | √ | √ |  |  |  |  |
| P5 = 1 0 0 - |  |  |  |  |  | √ | √ |  |
| P6 = 0 1 - 1 |  |  |  | √ | √ |  |  |  |
| P7 = - 1 1 1 |  |  |  |  | √ |  |  | √ |

The prime implicants  P1, P5 and P7 are essential. Removing these prime implicants and the minterms covered by them, we obtained the reduced covering table

| Prime implicants | MINTERMS |  |  |
| --- | --- | --- | --- |
|  | 4 | 5 |  |
| P2 = 0 - 0 0 | √ |  |  |
| P3 = - 0 0 0 |  |  |  |
| P4 = 0 1 0 - | √ | √ |  |
| P6 = 0 1 - 1 |  | √ |  |

Since P4 covers both minterms, the final cover is:

C  = {P1,P4,P5,P7} =

= {00-0,010-,100-,-111}

The minimal two-level implementation  corresponds to the function

fmin  =  x1 x2 x4 + x1  x2 x3 + x1 x2 x3 + x2  x3  x4 .

Using the Petrick function we obtain the same result.

Effectively, since we know that the essential prime implicants must belong to every minimal expression, it suffices to apply the Petick function to the reduced covering table. Thus, from the reduced covering table we obtain the following Petrick function:

P = (P2 + P4)(P4 + P6)  =  P4 + P2P6

P tells us that all the remaining minterms of the reduced covering table can be covered by P4. Therefore the minimal cover is composed of P1, P5,P7 and P4. This is the same result that we obtained before, namely,  C = {P1,P4,P5,P7} and

fmin  =  x1 x2 x4 + x1  x2 x3 + x1 x2 x3 + x2  x3  x4 .

___________________________________________________________________________

5.- 	Give the Shannon expansion of  F(A,B,C,D) = AB + AC + AD + BC + BD + CD

with respect to A,D.

_________________________________________________________________________

ANSWER

F(A,B,C,D) =     AD [F(0,B,C,0)] +

+A  D [F(0,B,C,1)] +

+   AD [F(1,B,C,0)] +

+   A  D [F(1,B,C,1)] .

F(A,B,C,D) =     AD [BC] +

+A  D [B + C] +

+   AD [B + C] +

+   A  D [ 1 ] .

F(A,B,C,D) =  AD [BC]  +A  D [B + C]  +  AD [B + C]  +   A  D [ 1 ] .

---------------------------------------END OF ASSIGNMENT  #1-----------------------------------