McGill University

Department of Electrical and Computer Engineering

Course: ECSE - 323							Winter  2008

Assignment #6

with solutions

TOPIC : Sequential Circuits

PART I: Sequential Circuits (Chapter 7, pages 380 and 381; Sections 7.1 (page 381) to Section 7.7 (page 398)).

1.-   Problem 7.4

___________________________________________________________________________

ANSWER

2.-  Given a T-type flip-flop, draw the circuit which produces a JK-type flip-flop.

a)  Derive clearly the logic circuit to produce  the desired flip-flop conversion

(see drawing below).

b)  Draw the complete circuit using minimum number of NAND gates.

_________________________________________________________________________

ANSWER

a)

Solving for T = f(Q; J,K)

| Q -> Q+ | J K | T |
| --- | --- | --- |
| 0 0 | 0 x | 0 |
| 0 1 | 1 x | 1 |
| 1 0 | x 1 | 1 |
| 1 1 | x 0 | 0 |

| JK Q | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 0 |

T = Q J + Q K

b)

Part II: Analysis of asynchronous sequential circuits (Chapter 9, page 578,

Sections 9.1 and 9.2 (pages578 to 590))

_______________________________________________________________

4.- Problem:

a)  Analyze the following circuit using state transition map by indicating with a circle within the map the stable states.

b)  For what value of the inputs x1, x2 oscillation occurs? (Reason your answer)

_________________________________________________________________

ANSWER

| a) x1x2 y | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 |

Y+ =x1 y +x1 x2 +y x2

Y+

b)  For inputs  x1=1  x2= 1 the circuit oscillates between states  0  and  1.

5.-

a)  Analyze the following circuit using state transition map by indicating with a circle within the map, the stable states.

b)  For what value of  x1, x2 oscillation occurs? (Reason your answer)

____________________________________________________________________________

ANSWER

| a) x1x2 y | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 1 | 0 |

Y+ = x1 x2 + x2 y

Y+

b) Since all rows have at least one circled state (stable state) there are no oscillations for any of the  4  possible inputs.

6.-

Consider the following combinational circuit with feedback which can be thought of as a basic memory element, M.

a)  Determine the stable states and the input values for which oscillation occurs by showing with a circle the corresponding entries in its state transition map.

b)  Using the given memory element, M, as a building block and additional NAND gates as needed, design and draw the circuit of  a JK-type memory element.

__________________________________________________________________________

ANSWER

a)

Q+ = xy + Q x y  + Qx                                                        next state map

| x y Q | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 |

Q+

For the input values  x = 0, y = 1 the circuit oscillates

For the other input values the circuit is stable.

b)

Solving for x and y as functions of  J, K and Q

we obtain

| Q -> Q+ | J K | X Y |
| --- | --- | --- |
| 0 0 | 0 * | 1 * |
| 0 1 | 1 * | 0 0 |
| 1 0 | * 1 | 1 0 |
| 1 1 | * 0 | 1 1 |

| J K Q | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 |

x =  Q + J

| J K Q | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

y = QK

with the condition   x y  =  0  to present oscillation

The resulting circuit is

**************END OF ASSIGNMENT #6 WITH SOLUTIONS************