McGill University

Department of Electrical and Computer Engineering

Course: ECSE - 323						                          Winter 2008

Assignment #7

with solutions

TOPIC: Registers and Counters

Registers (Section 7.8, page 399)

Problem 1

Design a circuit that adds 3 to the contents of a 4-bit register. Originally the register contains a 4-bit number  N  , where 000< N < 1100. After one single clock pulse the register contents should be N + 0011.

a) Explain briefly your design approach

b) Draw the complete circuit using JK-type edge-trigger flip-flops, full adders and  NAND gates as needed.

____________________________________________________________________________

ANSWER

a)  Load each flip-flop of the register with the output of a full-adder which has, respectively, as inputs the previous contents and the corresponding bit of the number 0011. Carry should ripple from one order to the next.

b)

Note that a D-type flip-flop can be built with a JK-type and an inverter thus

Problem 2

Design a 4-bit multifunction register that will behave according to the following table:

| A B | Function performed |
| --- | --- |
| 00 | Clear register |
| 01 | Complement register contents |
| 10 | Add 1 to register contents |
| 11 | Load register from input lines |

Where  A, B  are the mode select inputs. Use  4 x 1 multiplexers for mode selection

_______________________________________________________________________

ANSWER

Di = AB [ 0 ] +A B [Qi ] + AB [Si] + A B [Ii]

where  Si = Qi + Ci ( + means here arithmetic addition) for i = 0,1,2,3 with C0 = 1.

Counters

Problem 3

Using the 74X163 4-bit programmable counter, with synchronous CLEAR and LOAD inputs, whose block diagram is shown below, and the fewest number of gates possible, draw a circuit of a counter having  4  different counting sequences depending on the values of two mode selection variables  s1 s2 . The  4   counting sequences of the counter are the following

| Mode selection s1 s2 | Counting sequence |
| --- | --- |
| 00 | 0 to 15 |
| 01 | 0 to 7 |
| 10 | 0 to 9 |
| 11 | 0 to 12 |

Use  one  4 x 1  multiplexer for mode selection. Assume that the counter is self starting, meaning that the counter will always fall into the correct selected counting sequence after certain number of counts have elapsed.

____________________________________________________________________________

ANSWER:

Problem 4

Using the 74x163 4-bit programmable counter, with synchronous CLEAR and LOAD

Inputs, whose block diagram is shown below, and the fewest number of gates possible draw a SERLF-STARTING counter that will count following the sequence 5,6,7,8,9,10,5,6,7,8,9,10….

____________________________________________________________________________

ANSWER

Out-of-sequence detection logic

|  | Present state QD QC QB QA | Next state QD+ QC+ QB+ QA+ |
| --- | --- | --- |
| 0 | 0 0 0 0 | Load (0101) |
| 1 | 0 0 0 1 |  |
| 2 | 0 0 1 0 |  |
| 3 | 0 0 1 1 |  |
| 4 | 0 1 0 0 |  |
| 5 to 10 | Counting sequence | Counting sequence |
| 11 | 1 0 1 1 | Load (0101) |
| 12 | 1 1 0 0 |  |
| 13 | 1 1 0 1 |  |
| 14 | 1 1 1 0 |  |
| 15 | 1 1 1 1 |  |

_____

LOAD = QBQD QC +

+QAQD QC + QA QD QC + QB QDQC

************END OF ASSIGNMENT #7 WITH SOLUTIONS************