McGill University	DIGITAL SYSTEM DESIGN

Faculty of Engineering	ECSE-323B

FINAL EXAMINATION

WINTER  2004 (April 2004)

###### ANSWER  KEY

STUDENT NAME                            McGILL I.D. NUMBER

Examiner: Prof. J. Clark 				Associate Examiner:  Prof. Tal Arbel

Signature:						Signature:

Co-Examiner: Prof. M. Marin

Signature:

Date: April 21 , 2004                                         		Time: 2:00 pm

INSTRUCTIONS:     SEE NEXT PAGE.

McGILL UNIVERSITY

Department of Electrical and Computer Engineering

ECSE-323B									Winter 2003

FINAL EXAM

Question		Maximum Points	     Points Attained

1			 10

2			 15

3			 20

4			 15

5			 15

6			 15

- 15

- 15

- 15

- 20

- 15

- 10

Total		         180 points

Please write down your name: _____________________________________

Please write your student ID: ______________________________________

Instructions/Please read carefully!

This is a close book exam. No books or notes are allowed. You may use a standard calculator.

All work is to be done on the attached sheets and under no circumstance are booklets or loose sheets to be used. Write your name at the top of every sheet.

Read the question carefully. If something appears ambiguous, write down your assumption. The points have been assigned according to the formula that 1 point = 1 exam minute, so please pace yourself accordingly.

Your Name_______________________________________________________

Question 1 :  CMOS Circuit Technology (10 points)

Draw the pull-up and the pull-down networks of a CMOS circuit producing the AND-OR-INVERT function of 4 variables whose Boolean algebraic expression is

f (A,B,C,D) = AND-OR-INVERT (A,B,C,D) = NOT[OR[AND(A,B),AND(C,D)]].

Only true variables are available as inputs.

Marking scheme : 5 points for the correct pull-up network and 5 points for the correct pull-down network.

_________________________________________________________________________________________

ANSWER

PULL-UP NETWORK

PULL-DOWN NETWORK

Your Name__________________________________________________________________________________

Question 2 :Boolean Logic Theory (15 points)

Draw the circuit producing the function  f(A,B,C,D) = m(8,9,10,11,12,13,15) and using ONLY   the building block

g(x,y,z) = xy +  x z. Two such blocks are sufficient.

(5 points) 	a)   Explain  CLEARLY your approach to solve this problem.

(5 points) 	b)   Decompose f  in terms of  g.

(5 points)	c)   Draw the complete minimal circuit.

_________________________________________________________________

ANSWER

- We map on a K-map the function  f . If  f is not a Booean constant (0,1) or one of the input variables or their complement, it must be the output of a building block g. Now we try to express x, y, z  as functions of  A, B, C, D and determine if these functions are Boolean constants,  input variables or their complement. If any of them  are not, again,  they must be outputs of a bulding block  g. We iterate this process until input variables or their complement are found. If g is complete, the process converges. If not, there is no solution.

This process is illustrated by the following figure:

f (A,B,C,D) =  AB  + AC + A D = AB + A(C + D )

Comparing  f  with  g  , g(x,y,z) = xy +  x z,  we can write   f(A,B,C,D) = g(A,B, (C + D)) = g(A,B,g(1,C,D))

b)   f is expressed in terms of  g  thus

f(A,B,C,D) = g(A,B, (C + D)) = g(A,B,g(1,C,D)).

c) which produces the following circuit

Your Name______________________________________________________________________________

Question 3 : VHDL (20 points)

- (10 points) Using components, write a complete VHDL description of the AND-OR-INVERT circuit represented by the Boolean expression given in Question 1. Assume that design entities for AND, OR, and INVERT gates are already defined.

- (10 points) Using process blocks and arithmetic statements, write a complete VHDL description of a synchronous 10-bit counter circuit that counts in factorial sequence (assume that 0!=0).  Provide an asynchronous reset. Assume that the counter wraps back to zero when it reaches the maximum count value.

_________________________________________________________________________________________

a) f=NOT[OR[AND(A,B),AND(C,D)]].

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

ENTITY and_or_inv IS

PORT ( A,B,C,D : IN STD_LOGIC ;

f : OUT STD_LOGIC) ;

END and_or_inv ;

ARCHITECTURE Structure OF and_or_inv IS

COMPONENT AND2

PORT (x1,x2 : IN STD_LOGIC ;

y : OUT STD_LOGIC) ;

END COMPONENT;

COMPONENT OR2

PORT (x1,x2 : IN STD_LOGIC ;

y : OUT STD_LOGIC) ;

END COMPONENT;

COMPONENT INV

PORT (x : IN STD_LOGIC ;

y : OUT STD_LOGIC) ;

END COMPONENT;

SIGNAL I1, I2, I3 : STD_LOGIC;

BEGIN

A1: AND2 PORT MAP (x1=>A,x2=>B,y=>I1) ;

A2: AND2 PORT MAP (x1=>C,x2=>D,y=>I2) ;

O1: OR2 PORT MAP (x1=>I1,x2=>I2,y=>I3) ;

I1: INV PORT MAP (x=>I3,y=>f) ;

END Structure ;

Continue Question 3 on next page….

Your Name______________________________________________________________________________

Question 3:VHDL (20 points)

b)

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

USE ieee.std_logic_unsigned.all ;

entity factorial_counter is

port( clk, reset : in std_logic;

fact : out std_logic_vector(9 downto 0));

end factorial_counter;

architecture Q3 of factorial_counter is

signal icount : std_logic_vector(2 downto 0);

begin

count <= icount;

process(clk, reset)

begin

if reset = '1' then

icount <= "000";

fact <= "0000000000";

elsif clk='1' and clk'EVENT then

if icount = 6 then

icount <= "000";

fact <= "0000000000"

else

icount <= icount + 1;

if icount = "000" then

fact <= "0000000001";

else

fact <= fact * (icount + 1);

end if; -- if icount

end if; -- if icount

end if; -- if reset

end process;

end Q3;

Your Name__________________________________________________________________________________

Question 4 : Combinational Circuit Synthesis (15 points)

Design a 2-digit, base-3, adder circuit. This circuit adds 2 digits, A, B,  and produces outputs S = A + B (addition in base-3) and C (for Carry). Digits A, B and S are coded in binary such that, being A = {a1 , a2}, B = {b1 ,b2},

S = {s1 ,s2} the code assignment is

| A | {a1 , a2} | B | {b1 ,b2} | S | {s1 , s2} |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 0 | 0 | 0 0 | 0 | 0 0 |
| 1 | 0 1 | 1 | 0 1 | 1 | 0 1 |
| 2 | 1 0 | 2 | 1 0 | 2 | 1 0 |

Assume that assignment  {1,1} never occurs at the inputs.

(5 points) a)  Give the truth table of the outputs.

(5 points) b)  Draw the minimal NAND-NAND circuit  of the outputs.

(5 points) c)  Draw the circuit of the outputs using  only  4 x 1  multiplexers and the fewest number of gates.

_________________________________________________________________________________________

a)

| Identifier | A = {a1, a2} | B = {b1 ,b2} | S = {s1 , s2} | C (Carry) |
| --- | --- | --- | --- | --- |
| 0 | 0 0 0 | 0 0 0 | 0 0 0 | 0 |
| 1 | 0 0 0 | 1 0 1 | 1 0 1 | 0 |
| 2 | 0 0 0 | 2 1 0 | 2 1 0 | 0 |
| 3 | 0 0 0 | 3 1 1 | x xx | x |
| 4 | 1 0 1 | 0 0 0 | 1 0 1 | 0 |
| 5 | 1 0 1 | 1 0 1 | 2 1 0 | 0 |
| 6 | 1 0 1 | 2 1 0 | 0 0 0 | 1 |
| 7 | 1 0 1 | 3 1 1 | x xx | x |
| 8 | 2 1 0 | 0 0 0 | 2 1 0 | 0 |
| 9 | 2 1 0 | 1 0 1 | 0 0 0 | 1 |
| 10 | 2 1 0 | 2 1 0 | 1 0 1 | 1 |
| 11 | 2 1 0 | 3 1 1 | x xx | x |
| 12 | 3 1 1 | 0 0 0 | x xx | x |
| 13 | 3 1 1 | 1 0 1 | x xx | x |
| 14 | 3 1 1 | 2 1 0 | x xx | x |
| 15 | 3 1 1 | 3 1 1 | x xx | x |

Continue Question 4 on next page………..

Your Name______________________________________________________________________________

Question 4 : Combinational Circuit Synthesis(Continued)

b)

b)

Your Name______________________________________________________________________________

Question 4 : Combinational Circuit Synthesis(Continued)

c)

Your Name_______________________________________________________

Question 5 : Testing of Combinational Circuit(15 points)

For the following circuit

(1 point)		a)	Give the check points.

(2 points)	b)	Give the test for fault 3/1.

(7 points) 	c)	Give the minimal test sets.

________________________________________________________

- The check points are  2,3,4,5.

b)	f3/1 = x2 ;  f = x1x2  +  x2x3  ;  f3/1  f = [x2  (x1x2  +  x2x3) ] = x3  x2x1

Therefore, {010} is a test for 3/1.

c)

| Tests |  |  | Faults |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x1 | x2 | x3 | 2/0 | 2/1 | 3/0 | 3/1 | 4/0 | 4/1 | 5/0 | 5/1 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

After selecting the tets covering only one fault (essential), which are {010,011,110}, the fault table reduces to

| Tests |  |  | Faults |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x1 | x2 | x3 | 2/0 | 2/1 | 3/0 | 3/1 | 4/0 | 4/1 | 5/0 | 5/1 |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |

which produces the cover  {101}. Therefore, the minimum test set is {010,011,110,101}

You Name_____________________________________________________________________________

Question 6: Sequential circuits (15 points)

Analyze the following circuit

(5 points)	a) 	Prove with a state-transition-diagram (4 states) that this circuit is not a latch.

Modify it by replacing one EX-OR gate with an AND gate.

(5 points)	b) 	Give the characteristic and excitation equations (or tables) of this modified latch, called

EX-OR/AND latch.

(5 points)	c) 	Give the circuit of a JK-type latch using the EX-OR/AND latch circuit as a bulding block                                            and fewest additional gates.

____________________________________________________________________________________

a)

P+ = Q  x

Q+ = P  y

The state-transition-table will show the satable and unsateble sates:

| Present State | Next State: P+ Q+ |  |  |  |
| --- | --- | --- | --- | --- |
| P Q | x y = 00 | 01 | 10 | 11 |
| 0 0 | 00 | 01 | 10 | 11 |
| 0 1 | 01 | 11 | 00 | 01 |
| 1 0 | 01 | 00 | 11 | 10 |
| 1 1 | 11 | 10 | 01 | 00 |

If  P   = Q   (second and third row), then we observe that for xy = 01, 10, the present sate does not equal the next sate, and thus the circuit oscillates for these two inputs. Therefore x must be equal to  y to prevent oscillation. If this is the case, then from state  01, either xy = 00 or  xy = 11 will lock the circuit  into sate   01 permanently. And from  state  10,  xy = 00 will toggle the outputs locking them permanently to  01, and  xy = 11 will lock the outputs to 10.  Note that sate  10  cannot be reached from sate  01 !

If  P = Q  (which contradicts the properpy of a latch for which P = Q ) then the circuit is stable for x y = 00.

We conclude that the circuit in question is not a latch.

You Name_____________________________________________________________________________

Question 6: Sequential circuits (15 points) (Continues)

b)

Let us analyse this circuit

P+ = Q    x

Q+ = P  y

| Present State | Next State: P+ Q+ |  |  |  |
| --- | --- | --- | --- | --- |
| P Q | x y = 00 | 01 | 10 | 11 |
| 0 0 | 00 | 01 | 00 | 01 |
| 0 1 | 00 | 01 | 10 | 11 |
| 1 0 | 01 | 00 | 01 | 00 |
| 1 1 | 01 | 00 | 11 | 10 |

For this circuit to be a latch, P must be equal to  Q.  If this condition is satisfied,then  the circuit presents only one stable state, namely,   sate  01 and, therefore, it is useless as a bistable (memory) device.

c) In view of the conclusion reached in b) this circuit cannot be use as a building block to construct a JK-type latch. Thre is no solution to this part of the question.

Your Name___________________________________________________________________________________

Question 7: Counter Design (15 points)

Design a 3-bit self-starting UP/DOWN Johnson counter. When the UP/DOWN input is HIGH the counter counts up and when it is LOW the counter counts down. The state-transition-diagram of the counter is

(5 points)	a)	Give the state-transition-table of the counter

(5 points)	b)	Give the minimal sum-of-products of the excitation equations of the D-type flip-flops.

(5 points)	c)	Give the complete circuit using NAND gates.

Hint: Pay special attention to the self-starting requirement while constructing the state-transition-table.

| If the counter reaches an off-sequence sate, ten we assume the next sate to be 000. A B C | U/D = 0 | U/D = 1 |
| --- | --- | --- |
| 0 0 0 | 1 0 0 | 0 0 1 |
| 0 0 1 | 0 0 0 | 0 1 1 |
| 0 1 0 | 0 0 0 | 0 0 0 |
| 0 1 1 | 0 0 1 | 1 1 1 |
| 1 0 0 | 1 1 0 | 0 0 0 |
| 1 0 1 | 0 0 0 | 0 0 0 |
| 1 1 0 | 1 1 1 | 1 0 0 |
| 1 1 1 | 0 1 1 | 1 1 0 |
| A+ B+ C+ |  |  |

PRESENT      NEXT

STATE          STATE

C+ = U/D (AB) + C A B + UD(A B)

B+ = U/D (CA) + C A B + UD(C A )

A+ = U/D (C B) + C A B + UD(C B )

Your Name___________________________________________________________________________________

Question 7: Counter Design (15 points) (Continues)

Your name ________________________________________________________________________________

Question 8: Circuit Implementation Strategies/Logic arrays(15 points)

Using the PAL schematic shown in the  next page, which has 6 inputs and 5 outputs, two of which may be folded,

produce the following functions:

f1 = E  FD + E FC + B

f2 = D + C +E F B

f3 =   A BDE  F.

(7 points)	a)	Decompose the funtions so that they can be produced with the given PAL.

(8 points) 	b)	Complete the PAL program by marking clearly the required connections.

Not all connections are needed.

____________________________________________________________________________

a)

f1 = (E  F )(D + C ) + B

f2 = (D + C ) + (E F ) B

f3 =   A BD (E  F ).

(E  F  )  and  (D + C )   can be folded

We assume that all inputs and their complements are possible inputs to each AND gate.

Your Name_______________________________________________________________________________

Question 9: Finite State Machine Design(15 points)

Design a Mealey type Finite State Machine that detects the reception of at least 25 cents. There are three inputs, N for nickel, D for dime and Q for quarter. When the machine totalizes  25 cents or more, then  the output is asserted HIGH, and the machine remains in that state until it is manually reset, otherwise its output is asserted LOW.

The insertion of a coin is signaled by the associated signal going from low to high. The

signal will always go low before the next coin insertion.

(5 points)	a)  Give the minimal state-transition-diagram of this FSM.

(3 points)	b)  Give the ONE-HOT ASSIGNMENT state-transition-table .

(4 points)	c)  Give the complete circuit  using D-type flip-flops and AND, OR, NOT gates.

(3 points)	d)  Give the formula to compute the maximum clock rate of this  FSM.

____________________________________________________________________________

a) All transitions branches are asserted when RESET is not asserted. This condition garantees that when the external RESET signal is asserted to 1, all FFs, except the “0” flip-flop which is set to  1,  are set to  0. This condition is not shown in the State-transition-table(diagram) but it is shown in the final circuit.

b)

NEXT  STATE

| P.S. | N | D | Q | N,D,Q |
| --- | --- | --- | --- | --- |
| 0 | 5 | 10 | 25 | 0 |
| 5 | 10 | 15 | 25 | 5 |
| 10 | 15 | 20 | 25 | 10 |
| 15 | 20 | 25 | 2 | 15 |
| 20 | 25 | 25 | 25 | 20 |
| 25 | 25 | 25 | 25 | 25 |

The output of the circuit, which is not shown in the circuit drawing, corresponds to the function

OUTPUT = “0” Q + “ 5” Q + “10” Q + “15” (Q + D) + “ 20”(N + D + Q) + “25” (N +D +Q )

Continue Question 9 on next page  -------------

Your Name_______________________________________________________________________________

Question 9: Finite State Machine Design(15 points) (Continues)

d)  fclock = 1/T = 1/(tsu + tcc + tp)  where, tsu is the set-up time,  tcc  is he propagation delay of the combinational circuit and  tp is the propagation delay of the flip-flop.

Your Name_______________________________________________________

Question 10 : Design of Sequential Systems (20 points)

Using a synchronous Datapath/Controller approach, design a system that multiplies two 8-bit numbers using the Shift-and-Add technique.

- (5 points) Draw the block diagram for the system, showing all of the datapath elements and all control signals.

- (5 points) Draw the state-transition diagram for the controller.

- (10 points) Write a complete VHDL description of the system.

____________________________________________________________________________

A*B

a)

CONTROLLER

LD_SR1

SHL_SR1

LD_SR2

SHR_SR2

LD_AR

CLR_AR

CLR_CNT

EN_CNT

SR2_OUT

AREG

LD_AR

CLR_AR

RESET

COUNT7

CLOCK

ADDER

CLOCK

16 bit  SREG1

8bit SREG2

LD_SR1

SHL_SR1

LD_SR2

SHR_SR2

SR2_OUT

CLOCK

CLOCK

0      A

B

COUNT7

3-bit Counter

EN_CNT

CLR_CNT

S1

[LD_AR]

[EN_CNT]

[SHL_SR1]

[SHR_SR2]

CLOCK

SR2_OUT=1

COUNT7=0

b)

START

[CLR_AR]

[CLR_CNT]

[LD_SR1]

[LD_SR2]

DONE

SR2_OUT=1

COUNT7=1

SR2_OUT=1

COUNT7=0

SR2_OUT=0

COUNT7=0

S2

[EN_CNT]

[SHL_SR1]

[SHR_SR2]

COUNT7=1

SR2_OUT=0

RESET

SR2_OUT=0

COUNT7=0

Your Name_________________________________________________________________________________

Question 10 (continued)

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

entity shift_add_mult is

port( clk, reset : in std_logic;

A, B : in std_logic_vector(7 downto 0);

AB : out std_logic_vector(15 downto 0));

end shift_add_mult;

architecture Q10 of shift_add_mult is

type state_type is (START,S1,S2,DONE);

signal state : state_type;

begin

process(clk, reset)

begin

if reset = '1' then

state <= START;

elsif clk='1' and clk'EVENT then

case state is

when START =>

if SR2_OUT='1' then state<=S1;

else state<=S2' end if;

when S1 =>

if COUNT7='1' then state<=DONE;

elsif SR2_OUT='0' then state<=S2;

else state<=S1; end if;

when S1 =>

if COUNT7='1' then state<=DONE;

elsif SR2_OUT='1' then state<=S1;

else state<=S2; end if;

when DONE => state <= DONE;

end case;

end if; -- if reset

end process;

process(state) –- output logic

begin

CLR_AR <= '0'; -- provide defaults

LD_SR1 <= '0';

LD_SR2 <= '0';

LD_AR <= '0';

CLR_CNT <= '0';

EN_CNT <= '0';

SHR_SR2 <= '0';

SHL_SR1 <= '0';

case state is

when START =>

CLR_AR <= '1';

LD_SR1 <= '1';

LD_SR2 <= '1';

CLR_CNT <= '1';

when S1 =>

LD_AR <= '1';

EN_CNT <= '1';

SHR_R2 <= '1';

SHL_R1 <= '1';

when S2 =>

EN_CNT <= '1';

SHR_R2 <= '1';

SHL_R1 <= '1';

when DONE =>

end case;

end process;

end Q10;

Your Name__________________________________________________________________________________

Question 11 :Optimization of Finite State Machines (15 points)

Using the partitioning method reduce the following state transition table so that it contains the minimum number of states.

| Present State | Next State X = 0 X =1 |  | Outputs Z1 Z2 |  |
| --- | --- | --- | --- | --- |
| A | E | C | 0 | 0 |
| B | C | A | 0 | 0 |
| C | B | G | 0 | 0 |
| D | G | A | 0 | 0 |
| E | F | B | 1 | 0 |
| F | E | D | 0 | 0 |
| G | D | G | 0 | 0 |

(5 points)	First partition

(5 points)	Second partition

(5 points)	Final reduced transition table

____________________________________________________________________________

(A B C D E F G)

First partition: 		(A B C D F G) (E)  different outputs

Second partition: 		(AF) (BCDG) (E)

Third and final partition: 	(A) (F) (BD) (CG) (E)

Reduced table:

| Present State | Next State X = 0 X =1 |  | Outputs Z1 Z2 |  |
| --- | --- | --- | --- | --- |
| A | E | C | 0 | 0 |
| B | C | A | 0 | 0 |
| C | B | C | 0 | 0 |
| E | F | B | 1 | 0 |
| F | E | B | 0 | 0 |

Your Name_______________________________________________________

Question 12 : Testing of Sequential Circuits (10 points)

- (5 points) Draw a scan-path implementation of a 4-bit Johnson counter (i.e. a counter which has the count sequence 0000,0001,0011,0111,1111,1110,…).

- (2 points) What does the acronym JTAG stand for?

- (3 points) Briefly explain the idea behind the Boundary Scan test technique.

a)

TEST                     Q0                        Q1                        Q2                      Q3

SCAN_IN

D        Q

CLK

D        Q

CLK

D        Q

CLK   Q

D        Q

CLK

1

0

SCAN_OUT

CLOCK

b) JTAG = Joint Test Activities Group

c) The boundary scan technique inserts a register on every signal pin in a chip. These registers are connected together to form one long shift register, called the boundary scan register. The purpose of the boundary scan technique is to allow the connections on a printed circuit board to be tested. Test data can be shifted in, to drive the associated pin to a specific voltage, or the test data can be read out to read the voltage on a pc-board line that the pin is connected to.