EXPERIMENT 1 – DIGITAL FILTERING

Mathieu Perreault (260158758)

Logan Smyth (260179735)

Lab Group #2

- FUNCTIONAL REQUIREMENTS

The global deliverables in this experiment constitute a library of functions. In Part A of the experiment, the deliverable is coded in Assembly language. In Part B, the different functions are coded in embedded C. Together, they form the deliverable library.

The deliverables use a special format for data, called the 7-digit binary-coded decimals (BCD). Since it is a custom type made to fit within 32-bit values, a special format was specified in the requirements [1]:

A 32-bit representation example is:

• Bit 31 (MSB) is the sign bit: '0' for '+value', '1' for '-value',

• Bit 30 is the overflow flag: '0' for 'not overflown', '1' for 'overflown',

• Bit 29,28 are don't care bits,

• Bit 27-0 are data magnitude bits.

This format was implemented like specified above with the use of the unsigned long int masked as bcd32_t in our purposes. This special type implemented the list of requirements above.

1.1 Part A Requirements

Part A of this experiment had exactly one (1) deliverable.

Function: void bcdadd(bcd32_t* val1, bcd32_t* val2)

Purpose: This function should add two 32-bit 7 digit binary-coded decinal integers and store the result at the address of the 2nd result. It will cover adding two positive or negative values asn well as adding a negative and positive number.

Input: val1, val2 – pointers to two bcd integer values

Output: result = val1 + val2 – the final result is stored in the memory location passed in the second argument and will contain the result of the addition or signal overflow during addition

Special Cases/Error Conditions:

- positive + positive and negative + negative can potentially overflow and will return 0x40000000 if this is the case

- positive + negative cannot overflow but can result in either positive or negative values, which must be accounted for.

1.2 Part B Requirements

Part B of this experiment had multiple deliverables.  The final goal was to “implement functions in embedded C that perform simple filtering on a block of data”[1].

Function:  void bcdmult( bcd32_t* arg_a, bcd32_t* arg_b, bcd32_t* result)

Purpose:  This function should multiply two 32-bit binary-coded decimal (BCD) values and store the result in another 32-bit memory location in BCD form. It should cover all possible corner cases, including multiplication by/of negative numbers as well as overflown numbers.

Input: arg_a, arg_b – pointers to memory locations storing two positive or negative BCD-formatted values.

Output:  result = arg_a * arg_b – this memory location should contain the multiplication of the two BCD-coded input arguments. The result should be also coded in BCD form and properly indicated in the case of an overflow with the proper bit set (see representation above).

Special Cases/Error Conditions:

- Follow normal sign conventions – negative * negative = positive, negative*positive = negative, etc.

- If one or more of the operands are overflown (proper bit set), the multiplication will not take place

- If the result or any temporary computation has overflown the proper bit will be set and the function will return.

Function:  void fir(unsigned int filterOrder, bcd32_t* coeffs, bcd32_t* inputSamples, bcd32_t*

outputValue, unsigned int numInputs)

Purpose:  This function implements a digital Finite Impulse Response (FIR) filter on an input stream of data to produce a filtered response. Depending on the coefficients of the filter, this filter may act as a low-pass filter, for example. The formula dictating the filter output is the following [1]:

outputValue[n] = coeffs[0]*inputSamples[n] + ... + coeffs[N]*inputSamples[n-N]

Where n goes from N to numInputs and N = filterOrder -1.

Input:

- filterOrder – unsigned int indicating the order of the filter, also meaning the number of coefficients in the filter.

- coeffs – a pointer to a table of 32-bit BCD-formatted coefficients to the filter. There should be as many as the filterOrder.

- numInputs – an unsigned int indicating how many input samples will be input in the filter system.

- inputSamples – a pointer to a table of 32-bit BCD-formatted inputSamples to the filter. There should be as many as numInputs.

Output:

- outputValue – a pointer to the 32-bit location that will hold the BCD-formatted outputValue to the filter (one at a time).

Special Cases/Error Conditions:

- The function should assume that numInputs is always greater or equal to filterOrder.

- If one or more of the operands are overflown (proper bit set), outputValue will be overflown.

- IMPLEMENTATION

  - Part A Implementation

bcdadd: There are two primary cases that are covered in the algorithm. When adding numbers with the same sign, we much be able to recognize possible overflow. In order to be able to detect overflow in the negative values, we add only the magnitude of the values and watch for overflow into the 8th digit, and then replace the sign for the sign. If overflow is detected, then instead set the overflow bit.

When adding a negative and a positive number, we must also account for the fact that a negative or positive result may occur. To accomplish this, we compare the magnitudes of the values, and record whether or not the output will be positive or negative. If either input value is negative, the compliment of the value is taken and used during the addition, and then the sign is reapplied.

In order to take the compliment of the value, we loop though all seven digits and do (9 - #), and then add 1 to the value.

The addition itself is done by making use of the built in DADD assembly instruction that enables 32 bit decimal addition.

2.2 Part B Implementation

bcdmult: To execute in constant time, this function was implemented using a shifting technique. The algorithm takes one operand (let’s say, arg_a) and extract its digits turn by turn. For every digit, it does the bcdadd function on arg_b a number of times equivalent to the digit extracted. It then shifts the result according to the position of the extracted digit in arg_a. For example, multiplying 0x00000123 by 0x00000456 would perform the following:

(456+456+456) +(456+456)*10 + (456)*100, where the multiplications are in fact shifting left by a certain number of bits.

fir: This simple function was devised according to the specifications in “Functional Requirements,” above. It assumes that numInputs is at least equal to filterOrder. It separately does all the multiplications of the formula and adds the temporary results to a cumulative result. When the algorithm is done for every coefficient of the filter, the outputValue is updated.

- PERFORMANCE ANALYSIS

3.1 Cycles Consumption

| Function name | Min cycles | Max cycles | Average number of cycles (10 meas.) |
| --- | --- | --- | --- |
| bcdadd | 189 | 223 | 206 |
| bcdmult | 41003 | 56565 | 49143 |
| FIR (filterOrder = 3, to produces one outputValue) | 531031 | 542503 | 535312 |

Table 1 – Measure of cycles consumptions for different routines

3.2 Test Cases

BCDADD

| Input A | Input B | Expected Output | Real Output | Reason |
| --- | --- | --- | --- | --- |
| 0x00145312 | 0x07910234 | 0x08055546 | 0x08055546 | Standard addition, no OF |
| 0x00145300 | 0x87910200 | 0x87764900 | 0x87764900 | Add small positive to big negative |
| 0x03145300 | 0x07910200 | 0x40000000(overflow bit 30 set) | 0x40000000 | Positive overflowing |
| 0x83145300 | 0x87910200 | 0x40000000(overflow bit 30 set) | 0x40000000 | Negative overflowing |
| 0x80000345 | 0x00500400 | 0x00500055 | 0x00500055 | Small neg. + Large pos. |
| 0x00000345 | 0x80500400 | 0x00500055 | 0x80500055 | Small pos. + Large neg. |
| 0x07910234 | 0x00145312 | 0x08055546 | 0x08055546 | Same as 1st , swapped |

Table 2 – Test cases for the bcdadd routine

BCDMULT

| Input A | Input B | Expected Output | Real Output | Reason |
| --- | --- | --- | --- | --- |
| 0x00000123 | 0x00000456 | 0x00056088 | 0x00056088 | Standard mult, no overflow |
| 0x01111111 | 0x00000009 | 0x09999999 | 0x09999999 | Multiplying all the digits |
| 0x00000000 | 0x07237273 | 0x00000000 | 0x00000000 | Multiply by zero |
| 0x05000000 | 0x00000002 | 0x40000000 | 0x40000000 | Simple case of overflow |
| 0x80000001 | 0x01234567 | 0x81234567 | 0x81234567 | Multiply by -1 |
| 0x80000456 | 0x00000123 | 0x80056088 | 0x80056088 | Negative * positive |
| 0x80000456 | 0x80000123 | 0x00056088 | 0x00056088 | Negative * negative |

Table 3 – Test cases for bcdmult routine

FIR with filterOrder  = 3

| Coefficients | Input Samples | Expected Outputs | Real Outputs |
| --- | --- | --- | --- |
| 0x00000333 | 0x00000400 | -------- | -------- |
| 0x00000333 | 0x00000200 | -------- | -------- |
| 0x00000333 | 0x00000100 | 0x00233100 | 0x00233100 |
|  | 0x00000700 | 0x00333000 | 0x00333000 |
|  | 0x00000625 | 0x00474525 | 0x00474525 |

Table 4 – Test case for FIR routine standard 1/3 filter

FIR with filterOrder  = 3

| Coefficients | Input Samples | Expected Outputs | Real Outputs |
| --- | --- | --- | --- |
| 0x80000333 | 0x00000500 | -------- | -------- |
| 0x80000333 | 0x00000600 | -------- | -------- |
| 0x80000333 | 0x00000550 | 0x80549450 | 0x80549450 |
|  | 0x00000700 | 0x80616050 | 0x80616050 |
|  | 0x00000625 | 0x80624375 | 0x80624375 |

Table 5 – Test case for FIR routine negative coefficients

| Coefficients | Input Samples | Expected Outputs | Real Outputs |
| --- | --- | --- | --- |
| 0x00000333 | 0x80000500 | -------- | -------- |
| 0x00000333 | 0x80000600 | -------- | -------- |
| 0x00000333 | 0x80000550 | 0x80549450 | 0x80549450 |
|  | 0x80000700 | 0x80616050 | 0x80616050 |
|  | 0x80000625 | 0x80624375 | 0x80624375 |

Table 6 – Test case for FIR routine positive coefficients but negative input

| Coefficients | Input Samples | Expected Outputs | Real Outputs |
| --- | --- | --- | --- |
| 0x00000200 | 0x80000500 | -------- | -------- |
| 0x00000200 | 0x80000500 | -------- | -------- |
| 0x00000200 | 0x80000500 | -------- | -------- |
| 0x00000200 | 0x80000700 | -------- | -------- |
| 0x00000200 | 0x80000700 | 0x80580000 | 0x80580000 |
|  | 0x80000700 | 0x80620000 | 0x80620000 |
|  | 0x80000700 | 0x80660000 | 0x80660000 |

Table 7 – Test case for filterOrder 5 and 7 input samples

| Coefficients | Input Samples | Expected Outputs | Real Outputs |
| --- | --- | --- | --- |
| 0x00000333 | 0x00000500 | -------- | -------- |
| 0x00000333 | 0x00000500 | -------- | -------- |
| 0x00000333 | 0x00000500 | 0x00499500 | 0x00499500 |
|  | 0x00000700 | 0x00566100 | 0x00566100 |
|  | 0x00000700 | 0x00632700 | 0x00632700 |

Table 8 – Test case for the standard test in Experiment documentation

Appendix A: Algorithmic Diagrams

Drawing 1: Flow Chart of Addition Algorithm

Drawing 2: Flow Chart of the Multiplication Algorithm

Drawing 3: Flow Chart of the Filtering Algorithm

REFERENCES

[1] J.S. Chenard, “Lab 1: Assembly and Embedded C,” pp. 1-6, September 8th, 2009.

Appendix – Code

main.c

#include <stdio.h>

#include "bcd.h"//routines.h"

#include <cross_studio_io.h>

#include  <msp430x14x.h>

// A Main Program to call the subroutines

void main(void){  // main neither has arguments nor returns anything

int i;

bcd32_t multResult = 0x0;

bcd32_t bcdadd_tc[] = {

0x00145312, 0x07910234, 0x08055546,

0x87910200, 0x00145300, 0x87764900,  // carry test //this one doesn't work

0x03145300, 0x07910200, 0x40000000,  // sum test

0x83145300, 0x87910200, 0x40000000,  // subtraction test

0x80000345, 0x00000400, 0x00000055,  // small neg + large pos

0x80000345, 0x00000200, 0x80000145,  // big neg + small pos

0x00003480, 0x80345800, 0x80342320,  // small pos + big neg

0x00003000, 0x80000800, 0x00002200,  // big pos + small neg

0x89999990, 0x80000010, 0x40000000,  // overflow negative

0x09999990, 0x00000010, 0x40000000   // overflow positive

};

bcd32_t bcdmult_tc[] = {

0x00000123, 0x00000456, 0x00056088, /*mult that doesn't overflow*/

0x01111111, 0x00000009, 0x09999999, /*looking at all the digits*/

0x00000000, 0x07237273, 0x00000000,

0x05000000, 0x00000002, 0x40000000,

0x80000001, 0x01234567, 0x81234567,

0x80000456, 0x00000123, 0x80056088,

0x80000456, 0x80000123, 0x00056088

};

bcd32_t coefficients[] = {

0x00000333, 0x00000333, 0x00000333

};

bcd32_t inputSamples[] = {

0x80000500, 0x80000600, 0x80000550, 0x80000700, 0x80000625

};

for (i = 0;i < sizeof(bcdadd_tc)/sizeof(bcd32_t); i = i + 3) { //testing the addition

bcdadd(bcdadd_tc + i, bcdadd_tc+i+1);

if (bcdadd_tc[i+1] & 0x40000000) debug_printf("Addition overflowed ");

if (bcdadd_tc[i+1] == bcdadd_tc[i+2]) debug_printf("%dth case: gave 0x%1lX, expected 0x%1lX =>OK\n", i/3, bcdadd_tc[i+1], bcdadd_tc[i+2] );

else debug_printf("%dth case bcdadd => NOT OK, gave 0x%1lX\n",i/3, bcdadd_tc[i+1]);

}

for (i = 0; i < sizeof(bcdmult_tc)/sizeof(bcd32_t); i = i + 3) { //testing the multiplication

multResult = 0;

bcdmult(&bcdmult_tc[i], &bcdmult_tc[i + 1], &multResult); //result in multResult

//compare with multiplication results

if (multResult & 0x40000000) debug_printf("Multiplication overflowed\n");

if (multResult == bcdmult_tc[i+2]) debug_printf("bcdmult =>OK\n");

}

fir(3, coefficients, inputSamples, &multResult, sizeof(inputSamples)/sizeof(bcd32_t));

return;

}

// Comments explaining your bcdmult function

void bcdmult( bcd32_t* arg_a, bcd32_t* arg_b, bcd32_t* result){

int k = 0;

int m = 0;

int t = 0;

int aisneg = 0;

int bisneg = 0;

bcd32_t tempdigit = 0x0;

bcd32_t tempresult = 0x0;

//extract both sign bits to see the resulting

unsigned long int sign = ((*arg_a) & 0x80000000) ^ ((*arg_b) & 0x80000000); //if sign is 1 the number is negative

debug_printf("***Multiplying: 0x%1lX and 0x%1lX\n",*arg_a, *arg_b);

if (sign != 0) if (*arg_a & 0x80000000) aisneg = 1; else bisneg = 1;

*arg_a &= ~0x80000000;

*arg_b &= ~0x80000000; //clearing their MSBs

*result = 0;

if (*arg_a & 0x40000000 || *arg_b & 0x40000000){ //one of them is overflown

//debug_printf("One number is overflown. 0x%1lX or 0x%1lX\n",*arg_a, *arg_b);//what should we do ?

}

for (;k < 7; k++){

tempdigit = (*arg_a >> k*4) & 0xF ;

//debug_printf("tempDigit %d is 0x%1lX\n", k, tempdigit);

if (tempdigit){ //if arg_a has a digit in the kth position

tempresult = 0x0;

for (m = 0; m < tempdigit; m++){

//debug_printf("Adding 0x%1lX to tempresult 0x%1lX\n",*arg_b, tempresult);

bcdadd(arg_b, &tempresult);

}

if ( tempresult & 0xF0000000 ) { //overflown in the tempresult we are about to add

debug_printf("Addition/shift overflown 0x%1lX\n", tempresult);

*result = 0x40000000;

return;

}

for (t = 0; t < k; t++){

tempresult =tempresult << 4;

if ( tempresult & 0xF0000000 ) { //overflown in the tempresult we are about to add

debug_printf("Addition/shift overflown 0x%1lX\n", tempresult);

*result = 0x40000000;

return;

}

}

//debug_printf("tempResult %d is 0x%1lX\n", k, tempresult);

bcdadd(&tempresult,result);

}

}

if (sign != 0) {

*result |= 0x80000000;//if sign was one (negative number) just flip the sign bit

if (aisneg) *arg_a |= 0x80000000;

if (bisneg) *arg_b |= 0x80000000;

}

debug_printf(" gives 0x%1lX\n", *result);

}

// Comments explain your fir function.

void fir(unsigned int filterOrder, bcd32_t* coeffs, bcd32_t* inputSamples, bcd32_t* outputValue, unsigned int numInputs) {

int k = 0;

int m = 0;

int numIterations = numInputs - filterOrder +1;

bcd32_t tempFilterOutput = 0x0;

bcd32_t tempMultOutput = 0x0;

for (; k < numIterations; k++){ //let's say from 0 to (20-5) = 15

tempFilterOutput = 0;

for (m = 0; m < filterOrder; m++){ //from 0 to 4, let's say

tempMultOutput = 0;

bcdmult(coeffs + m,inputSamples + m,&tempMultOutput);

if (tempMultOutput & 0x40000000){

debug_printf("Overflow ");

}; //FIXME if tempMultOutput has overflown ???

bcdadd(&tempMultOutput, &tempFilterOutput); //result gets put in tempFilterOutput

if (tempFilterOutput & 0x40000000){

debug_printf("\nOverflow\n");

} //FIXME if tempFilterOutput has overflown ???

}

*outputValue = tempFilterOutput; //we should have the full output here

debug_printf("FIR output : 0x%1lX\n", *outputValue);

inputSamples += 1; //increment the inputSamples pointer by 1, going to next input

if (k ==0){} ; //dummy line

}

}

bcd.h

#define bcd32_t unsigned long int

/**

* Adds two 32-bit 7 digit signed BCD values by complimenting and using the built in DADD instruction

* Coded in assembly and linked in later

* Inputs: *arg1 and *arg2, two signed BCD vales

* Outputs: *arg2 => the sum of the two values

* Special Cases: When overflow is encountered, the 30th bit will be set to to a vlaue of one

*/

extern void bcdadd(bcd32_t *arg1, bcd32_t *arg2);

/**

* Multiplies two 32-bit binary-coded decimal (BCD) values and stores the result in another 32-bit memory location in BCD form. It should cover all possible corner cases, including multiplication by/of negative numbers as well as overflown numbers.

* Inputs: *arg_a and *arg_b, two signed BCD vales

* Outputs: *result, a signed BCD values that is the result of the multiplication, or overflow

* Special Cases: There is a good chance that overflow will occur and the overflow bit on the result will be set

*/

void bcdmult( bcd32_t* arg_a, bcd32_t* arg_b, bcd32_t* result);

/**

*  implements a digital Finite Impulse Response (FIR) filter on an input stream of data to produce a filtered response. Depending on the coefficients of the filter, this filter may act as a low-pass filter, for example.

* Inputs:  filterOrder - The order of the filter function

*          coeffs - The address of the coefficient values of the function

*          inputSamples - The address of the input signal discreet values

*          numInputs - The number of values stored in the inputSamples array

* Outputs: outputValue - The address to store the output values

* Special Cases: Assumes a numInputs value larger that the filterOrder value

*/

void fir(unsigned int filterOrder, bcd32_t* coeffs, bcd32_t* inputSamples, bcd32_t* outputValue, unsigned int numInputs);

bcdadd.s43

PUBLIC _bcdadd

RSEG CODE

_bcdadd

; Push registers onto stack to preserve state!

PUSH R4

PUSH R5

PUSH R6

PUSH R7

PUSH R8

PUSH R9

PUSH R10

PUSH R11

PUSH R12

PUSH R15

; Make space on stack

CLRC

SUB.W #8,SP

MOV #0, R9

; Move value from R15 into Stack

MOV.W 0(R15),0(SP)

MOV.W 2(R15),2(SP)

MOV.B 3(R15),R5

AND.B #0x0F,3(SP)

; Move value from R14 into Stack

MOV.W 0(R14),4(SP)

MOV.W 2(R14),6(SP)

MOV.B 3(R14),R4

AND.B #0x0F,7(SP)

; Initialize to 0, tracks if the final result should be complimented

MOV.W #0x0,R11

; If the value R14 is negative and R15 is positive, check if R14's magnitude is greater

; If so, then the final result will be negative, so it will need to be complimented

AND.B #0x80,R4

JZ notneg_r14

AND.B #0x80,R5

JNZ neg_r15

MOV #1, R9

; R14 neg, R15 pos

CMP.W 2(SP),6(SP); R14-R15

JL start_add

JNE mark_compliment1

CMP.W 0(SP),4(SP); R14-R15

JL start_add

mark_compliment1

MOV.W #0x1,R11

JMP start_add

neg_r15

notneg_r14

; If the value R15 is negative and R14 is positive, check if R15's magnitude is greater

; If so, then the final result will be negative, so it will need to be complimented

AND.B #0x80,R4

JNZ neg_r14

AND.B #0x80,R5

JZ notneg_r15

MOV #1, R9

; R14 pos, R15 pos

CMP.W 6(SP),2(SP); R15-R14

JL start_add

JNE mark_compliment2

CMP.W 4(SP),0(SP); R15-R14

JL start_add

mark_compliment2

MOV.W #0x1,R11

JMP start_add

notneg_r15

neg_r14

start_add

; If the 2 values are the same sign, then there is a possibility of overflow

; And skip code for complimenting, since the code will simply add magnitudes instead

CMP.B #1, R9

JNZ solve_same_sign_eq

; Initialize sign counter to 0

MOV.W #0,R7

; Check sign of R14 and complement the value if needed

AND.B #0x80,R4

JZ no_compl1

ADD.W #0x1,R7

PUSH R15

MOV.W SP,R8

ADD.W #6,R8

MOV.W R8,R15

CALL #_bcdcompliment

AND.B #0x0F,3(R15)

POP R15

no_compl1

; Check sign of R15 and complement the value if needed

AND.B #0x80,R5

JZ no_compl2

ADD.W #0x1,R7

PUSH R15

MOV.W SP,R8

ADD.W #2,R8

MOV.W R8,R15

CALL #_bcdcompliment

AND.B #0x0F,3(R15)

POP R15

no_compl2

JMP add_values

; Handles adding 2 values with the same sign

; Removes signs and marks for later replacing signs

solve_same_sign_eq

BIT.B R4,R5

JGE add_values

AND.B #0x0F, 3(SP)

AND.B #0x0F, 7(SP)

MOV #1,R9 ; Mark so sign will be replaced later

; All code converges again here

add_values

; Perform the bcd addition

CLRC

DADD.W 0(SP),4(SP)

DADD.W 2(SP),6(SP)

; Move final value into @R14 and clear sign/overflow bits

MOV.W 4(SP),0(R14)

MOV.W 6(SP),2(R14)

MOV.B 3(R14),R8

AND.B #0x0F,3(R14)

; Return space from stack

CLRC

ADD.W #8,SP

; Prepare sign bits for comparison

AND.B #0x80,R4

AND.B #0x80,R5

CMP.B R4,R5

JNZ no_invert_sign_after ; If both are same sign

CMP.B #1,R9

JNZ both_positive ; If we removed the sign

ADD.B #0x80,3(R14) ; restore the sign

both_positive

BIT.B #0x10,R8

JZ no_invert_sign_after ; If overflow!

; Just set the value to 0x40000000

MOV.W #0x4000,2(R14)

MOV.W #0x0000,0(R14)

no_invert_sign_after

; If the compliment flag was set at the beginning of the function

; then compliment the final value

CMP.W #0x1,R11

JNZ no_compl3

PUSH R15

MOV.W R14,R15

CALL #_bcdcompliment

POP R15

no_compl3

; Restore the program state to it's previous values

POP R15

POP R12

POP R11

POP R10

POP R9

POP R8

POP R7

POP R6

POP R5

POP R4

RET

; Subfunction to handle complimenting the bcd value passed into it

PUBLIC _bcdcompliment

RSEG CODE

_bcdcompliment

PUSH R4 ; Loop index

PUSH R5 ; Address backup

PUSH R6 ; Current byte lower

PUSH R7 ; Current byte upper shifted

PUSH R8 ; temp

PUSH R9 ; temp

MOV.W #3,R4

MOV.W R15,R5

; Loop over the 4 bytes of the BCD value

start

MOV.B @R15+,R6

; Pull out the upper and lower digit from the current byte into R6 and R7

MOV.B R6,R7

RRA.B R7

RRA.B R7

RRA.B R7

RRA.B R7

AND.B #0x0F,R6

AND.B #0x0F,R7

; Subtract the upper digit from 9 and shift back to correct location at MSB

MOV.B #9,R8

SUB.B R7,R8

ADD.B R8,R8

ADD.B R8,R8

ADD.B R8,R8

ADD.B R8,R8

; Subtract the digit lower digit from 9

MOV.B #9,R9

SUB.B R6,R9

; Add the 2 digits together and push onto the stack for later use

ADD.B R8,R9

PUSH R9

DEC.W R4

JGE start

MOV.W R5,R15 ; Restore dest address

; Back up the sign bits of the input

MOV.B 3(R15),R8

; Pop digits off of stack and set values in return address

POP R4

POP R5

POP R6

POP R7

MOV.B R7,0(R15)

MOV.B R6,1(R15)

MOV.B R5,2(R15)

MOV.B R4,3(R15)

; Add one to the complimented values

CLRC

DADD.W #1,0(R15)

DADD.W #0,2(R15)

; Clear sign and overflow bits

AND.B #0x0F,3(R15)

; toggle the sign bit from it's previous state

AND.B #0x80,R8

XOR.B #0x80,R8

CLRC

ADD.B R8,3(R15)

; Restore program context

POP R9

POP R8

POP R7

POP R6

POP R5

POP R4

RET

END